.heTES002  V4.0                                           13/03/85

#WINCHESTER DISK DATA INTEGRITY TEST#

#INTRODUCTION#

Thi� progra� provide� � dat� integrit� chec� o� � wincheste� dis� �

drive� I� i� � no� destructiv� tes� bu� doe� allo� th� ba� bloc� �

tabl� t� b� modifie� a� th� en� o� th� tes� i� desired� Th� �

winchester disk should be backed up before running this program.

#OPERATION#

Progra� i� supplie� o� � singl� side� dis� whic� autoboot� t� �

displa� the following screen:

TES002 - configuration information

==================================

machine type      (details)

controller board  (   "   )

drive 1           (   "   )

drive 2 (if connected) ( "  )

TES002 - carrying out preliminary controller board test

Testing controller board registers... (Pass or Fail message)

Testing ram addressing...                     "     "

Testing ram data retention...                 "     "

NOTE:

If 2 drives are connected the program prompts:

Please give number of drive to test 1<cr> or 2<cr>

A� thi� poin� th� progra� attempt� t� rea� th� heade� labe� an� �

if successful prompts:

Press <cr> to see bad sector information

If the program was not successful it will display:

*** TES002 - failed to read valid label sector from disk ***

Please enter device information...

Give number of cyls in decimal ?

Give number of heads in decimal ?

Give sectors per track in decimal ?

Once this info has been keyed in the program prompts:

Press <cr> to see bad sector information

Pressing <cr> makes the program display:�

<C� AC�     Wincheste� dis� dat� integrit� chec�   TES00�  Ve� x.x

CYLINDER       HEAD      SECTOR         VIRTUAL

1)   xxx            x         x              xxxx

Ditto for all bad blocks

End of bad block display ... press <cr> to continue

Pressing <cr> makes the program display:

A cyclic test will keep looping until you stop it

A single pass test goes across the disk once only

Either test may be stopped by typing 's'

Type 1<cr> for a cyclic test

Type 0<cr> for a single pass test

Typing 1 will continue the test with the  following display:

cyclic test - type letter 's' to end          Loop times =  xx

X Mb winchester Checking head  x   Cylinder xxx

Th� scree� wil� contai� � lis� o� th� ba� sector� foun� an� � �

percentage fail rate.

A line at the bottom of the screen reports how many errors.

A� th� en� o� th� tes� (o� afte� pressin� S�� th� scree� wil� �

displa� th� numbe� o� ba� block� detecte� an� als� th� numbe� o� �

ba� block� alread� i� th� table.

Th� use� i� aske� whethe� h� want� t� ad� an� ba� block� t� th� �

tabl� (1=Yes 0=No) I� 0 th� progra� finishes�

I� � th� progra� display� eac� ne� ba� bloc� an� ask� whethe� �

thi� i� t� b� adde� t� th� tabl� (1/0).

Note�� Canno� ad� ba� block� 0-25� (Operatin� syste� need� these� �

and cannot add any more bad blocks once the table has 127 entries.

Th� progra� firs� ask� permissio� an� i� give� wil� ad� ba� �

block� t� th� tabl� an� the� promp� th� use� t� pres� rese� t� �

en� th� program.

NOTE: USE OF FAILRATE INFORMATION

0 - 16  Ignore error

17 - 90  Soft error

90 - 100  Hard error

#DESCRIPTION OF TESTS#�

Th� progra� begin� b� testin� th� dis� controlle� boar� a� �

follows:

Interfac� - Tes� o� address� dat� an� contro� line� betwee� �

apricot and controller board.

Buffe� - Tes� o� addres� an� dat� line� fo� th� 8� o� onboar� �

static ram.

Interface-See� -  Tes� tha� th� dis� drive/controlle� boar� �

combinatio� ca� successfull� see� t� th� correc� trac� o� th� �

drive.

Interface-Restor� - Tes� tha� � restor� comman� move� th� head� �

to track 0.

Th� progra� nex� attempt� t� rea� th� heade� labe� o� th� dis� �

and if successful displays the bad block information.

I� unsuccessfu� th� progra� ask� th� use� fo� th� siz� o� th� �

winchester and then continues.

Th� progra� attempt� t� rea� dat� fro� ever� secto� o� th� dis� �

checkin� th� WD101� erro� flag� afte� eac� read� I� a� erro� fla� �

i� se� th� progra� retrie� 5� times.Thi� i� the� displaye� o� �

screen as a percentage fail rate.

Afte� readin� th� entir� dis� th� progra� allow� th� use� t� �

modify the bad block table if desired.

NOTE� Th� use� formatte� (DISK� mus� b� use� afte� TES00� i� an� �

ba� block� hav� bee� found� Thi� i� becaus� TES00� doe� no� hav� �

the facility to correct the FATs.

----------------------

###############################################################################