Mouse Driver

**************

1. Overview

2. Applications interest

2.1 Infra-red (User int F5 hex)

2.2 Serial (User int FA hex)

2.3 Language interfaces

2.4 Functions

3. Systems interest

3.1 Driver operation

1. Overview

The Mouse Driver supports both the Infra-red and the Microsoft

Serial Mouse devices.

The Mouse is not supported by the Control Device and does not

form part of the resident BIOS - it is a separate installable

Device driver.

Application software interfaces with the Mouse Driver via

function calls to interrupt 33 hex or Far Calls in Basic as

described in the section Language interfaces. The functions

are described in a later section.

Data from the Mouse is received in Hamming encoded form by

the Hardware interrupt drivers. It is decoded and checked for

errors and then directed to either the Serial driver

(Microsoft) or Keyboard driver (Infra-red).

The Drivers filter data for the Mouse and invoke their

respective User interrupts.

The User interrupts F5 hex and FA hex are implemented within

the Mouse driver. The format of the Mouse data in each case

is discussed in the following sections to enable Applications

to interrupt and/or replace the Driver handling.

The Driver performs the function of collecting data from the

Mouse such as whether a button has been pressed, if so how

many times, and whether the Mouse has moved. It also performs

the function calls from the application such as Mouse position

and Button status or change Cursor characteristics.

2. Applications interest

2.1 Infra-red (User interrupt F5 hex)

The Mouse sends a packet of data in the following format:

---------------------------------

|F|E|D|C|B|A|9|8|7|6|5|4|3|2|1|0|

---------------------------------

|_____| | | | | |_____________|

|    | | | |        |

|    | | | |        |__________ X or Y code

|    | | | |

|    | | | |___________________ Axis (X=0 Y=1)

|    | | |_____________________ Switch 1 (0=off)

|    | |_______________________ Switch 2 (0=off)

|    |_________________________ 1 (Mouse)

|

not used

The Keyboard driver receives the data in the format

illustrated above in the AX register and if bit B hex is set

it executes a User interrupt F5 hex.

The Mouse sends a word of data whenever a change of state

occurs, i.e it has moved or a button has been pressed.

The X and Y data values are two's compliment integers

representing the relative displacement of the Mouse in the

respective coordinate since the last data transmission. A

positive value represents movement either upwards or right

whereas a negative value represents movement downwards or to

the left in the X and Y coordinates respectively.

Bit 8 determines the plane.

2.2 Serial (User int FA hex)

The Serial Mouse sends a sequence of 3 bytes which are

intercepted by the Serial Driver which executes an FA hex

interrupt, if the Mouse is enabled, with each byte in the AL

register in the following format:

-----------------

Byte 1 :  |7|6|5|4|3|2|1|0|   Status

-----------------

| | | | | | | |____ X6 data

| | | | | | |______ X7 data

| | | | | |________ Y6 data

| | | | |__________ Y7 data

| | | |____________ Right button (1=on)

| | |______________ Left button (1=on)

| |________________ Sync bit =1 (first byte)

|

not used

-----------------

Byte 2 :  |7|6|5|4|3|2|1|0|   X data

-----------------

| | |_________|

| |      |

| |      |________ X0-5 data (bit 0=X0...)

| |_______________ Sync bit = 0

|

not used

-----------------

Byte 3 :  |7|6|5|4|3|2|1|0|   Y data

-----------------

| | |_________|

| |      |

| |      |________ Y0-5 data (bit 0=Y0 ...)

| |_______________ Sync bit = 0

|

not used

The X and Y data values are two's compliment integers

representing the relative displacement of the Mouse in the

respective coordinate since the last data transmission. A

positive value represents movement either upwards or right

whereas a negative value represents movement downwards or to

the left in the X and Y coordinates respectively.

2.3 Language interfaces

The Driver supports entry from 8086 machine code via interrupt

33 hex or from Basic via a long call. The Driver expects 4

integer parameters as described in the section Functions. The

calls are:

Basic : The Driver has an inbuilt interface to Basic which

is located at the beginning of the interrupt routine

+ 2 bytes. Applications may pick up the address as

follows:

10 DEF SEG=0

20 MSEG=PEEK(51*4+2)+(256*PEEK(51*4+3))    'segment

30 MOUSE=(PEEK(51*4)+2)+(256*PEEK(51*4+1)) 'offset+2

40 IF MSEG < &HF000 THEN 60                'in RAM ?

50 PRINT "Mouse driver not installed": END

60 DEF SEG=MSEG     'to call the MOUSE driver

The calling sequence requires 4 parameters as

defined in the Functions detailed below. All 4

parameters must be included, whether needed or not,

otherwise the stack will be corrupted.

The example continues with a set cusor position

command:

70 M1%=4       ' function

80 M3%=320     ' horizontal coordinate

90 M4%=100     ' vertical

The function is executed by:

100 CALL MOUSE(M1%,M2%,M3%,M4%)

Note: M2% is included although it is not required.

8086 : Calls to the Mouse driver are executed in machine

code by loading the registers AX,BX,CX and DX with

the parameter values and then invoking a software

interrupt. The example as given above would be coded

as follows:

MOV AX,4       ; function

MOV CX,320     ; horizontal coordinate

MOV DX,100     ; vertical coordinate

INT 51         ; invoke the call

2.4 Functions

The functions implemented within the driver are as follows:

Number    Function

0       Mouse install flag and reset

1       Show cursor

2       Hide cursor

3       Get Mouse position and Button status

4       Set Mouse cursor position

5       Get Button press information

6       Get Button release information

7       Set Minimum and Maximum Horizontal position

8       Set Minimum and Maximum Vertical position

9       Set Graphics cursor block

A       Set Text cursor

B       Read Mouse motion counters

C       Set user-defined subroutine input mask

D       Light Pen Emulation (not implemented)

E       Light Pen Emulation (not implemented)

F       Set Mickey/Pixel Ratio

Full details of each function are given in the following

pages.

Function 0 : MOUSE INSTALLED FLAG RESET

Input            Output

M1% = 0            M1% = mouse status

M2% = number of buttons

(always two)

Returns the current status of the mouse hardware and software.

The mouse status is 0(false), if the mouse hardware and

software are not installed, and is -1(true), if the hardware

and software are installed.

The function also resets the mouse driver to the following

default parameters:

Function                           Parameter

Cursor position                      Screen centre

Internal cursor flag                  -1

Graphics cursor shape/HOT SPOT       ARROW/(-1-1)

Text cursor                          Inverted box

User defined CALL MASK               All zeroes

Light Pen emulation mode             Enabled

Mickey to Pixel ratio (horizontal)   8 TO 8

Mickey to Pixel ratio (vertical)     16 TO 8

Min/max cursor position (horizontal) 0/639

Min/max cursor position (vertical)   0/255        ?????

Example

To ensure that mouse hardware/software is installed and to

reset to default values:

200 M1% = 0

210 CALL MOUSE (M1%,M2%,M3%,M4%)

220 IF NOT (M1%) THEN PRINT " Mouse not installed":END

Function 1: SHOW CURSOR

Input              Output

M1% = 1               -

Increments the internal cursor flag and, if the flag is 0,

displays the cursor on the screen.  The cursor tracks the

motion of the mouse, changing position as the mouse changes

position.

The current value of the internal cursor flag depends on the

number of calls that have been made to Function 1 and Function

2.  See the section in this chapter "The Internal Cursor

Flag."  The default flag value is -1, so after a reset a

call to Function 1 will display the cursor.

Example

200 M1% = 1

210 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 2 : HIDE CURSOR

Input        Output

M1% = 2        -

Removes the cursor from the screen and decrements the internal

cursor flag.  The cursor, even though hidden, still tracks the

motion of the mouse, changing position as the mouse changes

position.

Use this function before modifying any portion of the screen

containing the cursor.  This prevents the cursor from possibly

affecting the data written to the screen.

Remember that each call to this function decrements the

internal cursor flag.  Each call to this function will require

a subsequent call to Function 1 to restore the flag to its

previous value.  See the section in this chapter, "The

Internal Cursor Flag."

Example

200 M1% = 2

210 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 3 : GET MOUSE POSITION AND BUTTON STATUS

Input               Output

M1% = 3               M2% = Button STATUS

M3% = Cursor position

(Horizontal)

M4% = Cursor position

(Vertical)

Returns the state of the left and right buttons and the

horizontal and vertical positions of the cursor.  The button

status is a single integer value.  Bits 0 and 1 represent the

left and right buttons, respectively.  A bit is 1 if a button

is down, and 0 if up.

The cursor positions are always within the range of minimum

and maximum values of the virtual screen.  See the section in

this chapter, "The Virtual Screen."

Example

200 'Get current  cursor positions, check button status

210 M1% =3

220 CALL MOUSE (M1%,M2%,M3%,M4%)

220 IF M2% AND 1 THEN PRINT "Left button down"

230 IF M2% AND 2 THEN PRINT "Right button down"

Function 4: SET MOUSE Cursor position

Input                                     Output

M1% =4

M3% = (horizontal) new cursor position

M4% = (vertical) new cursor position

Sets the cursor to the specified horizontal and vertical

screen positions.  The new values must be in the horizontal

and vertical ranges of the virtual screen.  If the screen is

not in high resolution mode, the values are rounded to the

nearest horizontal or vertical values permitted for the

current screen mode.  See the section in this chapter, "The

Virtual Screen"

Example

Assume that HMAX and VMAX contain the maximum horizontal and

vertical positions values for the vertical screen.  To set the

cursor to the centre of the screen:

200 ' Put cursor in centre of screen

210 M1% = 4

220 M3% = INT(HMAX/2)

230 M4% = INT(VMAX/2)

240 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 5: GET BUTTON PRESS INFORMATION

Input               Output

M1% = 5               M1% = button status

M2% = button          M2% = count of button presses

M3% = cursor (horizontal) at last press

M4% = cursor (vertical) at last press

Returns current button status, a count of button presses since

the last call to this function and the horizontal and vertical

position of the cursor at the last press of the button.

The parameter M2% specifies which button is checked.  If set

to 0 then the left button is checked.  If 1, the right button

is checked.

The button status is a single integer value.  Bits 0 and 1

represent the left and right buttons, respectively.  A bit is

1 if a button is down, and 0 if up.

The count of button presses is always in the range 0 to 32767;

overflow is not detected.  The count is set to 0 after the

call.

The horizontal and vertical values are in the ranges defined

by the virtual screen.  Note that these values represent the

cursor position when the button was last pressed and do not

represent the cursor's current position.

Example

200 ' Get cursor position at last button press.

210 M1% = 5

220 M2% = 0  Left Button

230 CALL MOUSE(M1%,M2%,M3%,M4%)

240 IF (M1% AND 1) THEN PRINT "Left button down."

Function 6: GET BUTTON RELEASE INFORMATION

Input               Output

M1% = 6               M1% = button status

M2% = button          M2% = count of button releases

M3% = cursor (horizontal) at last release

M4% = cursor (vertical) at last release

Returns current button status, a count of button releases

since the last call to this function, and the horizontal and

vertical position of the cursor at the last release of the

button.

The parameter M2% specifies whcih button is checked.  If set

to 1, the right button is checked.

The button status is a single integer value.  Bits 0 and 1

represent the left and right buttons, respectively.  A bit is

1 if a button is down and 0 if up.

The count of button releases is always in the range 0 to

32767; overflow is not detected.  The count is set to zero

after the call.

The horizontal and vertical values are in the ranges defined

by the virtual screen.  Note that these values represent the

cursor position when the button was last released and do not

represent the cursor's current position.

Example

200 ' Get cursor position at last button release.

210 M1% = 6

220 M2% = 1 right button

230 CALL MOUSE (M1%,M2%,M3%,M4%)

240 IF (M1% AND 2) THEN PRINT "Right button down."

Function 7 : SET MINIMUM AND MAXIMUM HORIZONTAL POSITION

Input                       Output

M1% = 7                        -

M3% = minimum position

M4% = maximum position

Sets the minimum and maximum horizontal cursor positions on

the screen.  Subsequent cursor motion is restricted to the

specified area.  The minimum and maximum values are defined by

the virtual screen. (See the section in this chapter, "The Virtual Screen.")

If the cursor is outside the area when the call is made, it

moves to just inside the area.  If the minimum value is

greater than the maximum, the two values are swapped.

Example

200 ' Limit cursor to horizontal positions below 150

210 M1% = 7

220 M3% = 0

230 M4% = 150

240 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 8: SET MINIMUM AND MAXIMUM VERTICAL POSITION

Input               Output

M1% = 8

M3% = Minimum position

M4% = Maximum position

Sets the minimum and maximum vertical cursor position on the

screen.  Subsequent cursor motion is restricted to the

specified area.  The minimum and maximum values are defined by

the virtual screen.  (See the section in this chapter, "The

Virtual Screen.')

If the cursor is outside the area when the call is made, it

moves to just inside the area.  If the minimum value is

greater than the maximum, the two values are swapped.

Example

200 ' Limit cursor to vertical positions between 100

210 ' and 150

220 M1% = 8

230 M3% = 100

240 M4% = 150

250 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 9: SET GRAPHICS CURSOR BLOCK

Input                                      Output

M1% = 9                                          -

M2% = Cursor HOT SPOT (Horizontal)

M3% = Cursor HOT SPOT (Vertical)

M4% = Pointer to Screen and Cursor MASKS

Defines the shape, colour, and centre of the cursor when in

graphics mode.

The function uses the values found in the screen mask and

cursor mask to build the cursor shape and color.  (See the

section in this chapter, "The Graphics Cursor")  To pass the

screen mask and cursor mask in BASIC, assign their values to

an integer array and use the first element of the array as the

fourth parameter in the call (see example).  To pass the

screen and cursor masks assembly languae, assign their values

to two contiguous arrays and pass the address of the first

array in register DX.  Be sure to load the segment address of

the arrays in the ES register before making the call.

The cursor hot spot values must define one pixel within the

cursor .  (See the section in this chapter, "The Graphics

Cursor Hot Spot.") The values must be within the range  -16 to

16.

Example

To define a cursor in high-resoloution graphics mode, first

define the values to the cursor array and then make the

call:

200  . DEFINE THE SCREEN MASK

210 CURSOR(0,0)=&HFFFF          '1111111111111111

220 CURSOR(1,0)=&HFFFF          '1111111111111111

230 CURSOR(2,0)=&HFFFF          '1111111111111111

240 CURSOR(3,0)=&HFFFF          '1111111111111111

250 CURSOR(4,0)=&HFFFF          '1111111111111111

260 CURSOR(5,0)=&HFFFF          '1111111111111111

270 CURSOR(6,0)=&HFFFF          '1111111111111111

280 CURSOR(7,0)=&HFFFF          '1111111111111111

290 CURSOR(8,0)=&HFFFF          '1111111111111111

300 CURSOR(9,0)=&HFFFF          '1111111111111111

310 CURSOR(10,0)=&HFFFF         '1111111111111111

320 CURSOR(11,0)=&HFFFF         '1111111111111111

330 CURSOR(12,0)=&HFFFF         '1111111111111111

340 CURSOR(13,0)=&HFFFF         '1111111111111111

350 CURSOR(14,0)=&HFFFF         '1111111111111111

360 CURSOR(15,0)=&HFFFF         '1111111111111111

400 ' DEFINE CURSOR MASK

410 CURSOR(0,1)=&H8000          '1000000000000000

420 CURSOR(1,1)=&HE000          '1110000000000000

430 CURSOR(2,1)=&HF800          '1111100000000000

440 CURSOR(3,1)=&HFE00          '1111111000000000

450 CURSOR(4,1)=&HD800          '1101100000000000

460 CURSOR(5,1)=&H0C00          '0000110000000000

470 CURSOR(6,1)=&H0600          '0000001100000000

480 CURSOR(7,1)=&H0300          '0000000110000000

490 CURSOR(8,1)=&H0000          '0000000000000000

500 CURSOR(9,1)=&H0000          '0000000000000000

510 CURSOR(10,1)=&H0000         '0000000000000000

520 CURSOR(11,1)=&H0000         '0000000000000000

530 CURSOR(12,1)=&H0000         '0000000000000000

540 CURSOR(13,1)=&H0000         '0000000000000000

550 CURSOR(14,1)=&H0000         '0000000000000000

560 CURSOR(15,1)=&H0000         '0000000000000000

600 ' DEFINE CURSOR SHAPE, COLOUR AND CENTRE

610 M1% = 9

620 M2% = 0    ' Horizontal HOT SPOT

630 M3% = 0    ' Vertical HOT SPOT

640 CALL MOUSE (M1%,M2%,M3%,CURSOR(0,0))

Function 10 : SET TEXT CURSOR

Input                              Output

M1% = 10

M2% = Cursor select

M3% = SCREEN MASK value/Scan line start

M4% = CURSOR MASK value/Scan line stop

Selects the software or hardware text cursor.  If the software

text cursor is selected, this function defines the character

attributes of the cursor when in text mode.  If the hardware

text cursor is selected, this function defines the first and

last scan lines to be shown on the screen.

The value of the parameter M2% selects the cursor type.  If

the value is 0, the software text cursor is selected.  If the

value is 1, the hardware text cursor is selected.

If the software text cursor is selected, the parameters M3%

and M4% must specify the screen and cursor masks.  These masks

define the attributes of a character when the cursor is over

it.  (See the section in this chapter, "The Software Text

Cursor.")  The mask values depend on the display adapter in

your computer.

If the hardware cursor is selected, the parameters M3% and M4%

must contain the line numbers of the first and last scan line

in the cursor to be show on the screen.  (See the section in

this chapter, "The Hardware Text Cursor.")  The line numbers

depend on the display adapter in your computer.

Example

To create a text cursor that inverts the foreground and

background colors:

200  M1% = 10

210  M2% = 0        ' SELECT TEXT CURSOR

220  M3% = &HFFFF   ' SCREEN MASK

230  M4% = &H7700   ' CURSOR MASK

240  CALL MOUSE (M1%,M2%,M3%,M4%)

Function 11 : READ MOUSE MOTION COUNTERS

Input               Output

M1% = 11              M3% = Count (Horizontal)

M4% = Count (Vertical)

Returns the horizontal and vertical mickey count since the

last call to this function.  The mickey count is the distance

in 1/100 inch increments that the mouse has moved.  (See the

section in this chapter, "The Mouse Unit of Distance: The

Mickey.")

The mickey count is always within the range - 32768 to 32767.

A positive horizontal count specifies a motion to the right.

A positive vertical count specifies a motion to the bottom of

the screen.  Overflow is ignored.

The mickey count is set to 0 after the call is completed.

Example

200 ' GET THE MICKEY COUNT

400 M1% = 11

500 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 12: SET USER-DEFINED SUBROUTINE INPUT MASK

Input                                 Output

M1% = 12

M3% = CALL MASK

M4% = Address offset to subroutine

Sets the call mask and subroutine address for the mouse

software interrupts.  The mouse software interrupts

automatically stop execution of your program and call the

specified subroutine whenever one or more of the conditions

defined by the call mask occur.  On completion of the

subroutine, your program will continue execution at the point

of interruption.

The call mask, a single integer value, defines which

conditions will cause an interrupt.  Each bit in the call mask

corresponds to a specific condition as shown here:

Mask bit      Condition

0           Cursor position

1           Left Button pressed

2           Left Button released

3           Right Button pressed

4           Right Button released

5-15        Not used.

To enable an interrupt for a given condition, set the

corresponding call mask bit to 1 and pass the mask parameter

M3%.  To disable a condition, set the corresponding bit to 0

and pass the mask.  All conditions are automatically disabled

by Function 0.

When the mouse software makes a call to the subroutine, it

loads the following information into the CPU registers:

Register    Information

AX            CONDITION MASK (similar to the CALL MASK

except a bit is set only

if the condition has

occurred)

BX            Button state

CX            Cursor position (Horizontal)

DX            Cursor position (Vertical)

To use this function with the BASIC interpreter, load an

assembly language subroutine into memory (use the same segment

as the BASIC interpreter), assign the entry address of the

subroutine to an integer variable, and pass this varible to

Funtion 12 as the fourth parameter.

To use this function in assembly language, load the ES

register with the subroutine's segment address, and load the

DX register with the subroutine's offset.

Example

Assuming that a subroutine has been loaded into

memory and that the integer variable SKETCH has been

assigned the subroutine's entry address, use the

following statements to set up calls on any press of

the left button.

200 ' CALL SUBROUTINE SKETCH ON LEFT BUTTON PRESS

210 M1% = 12

220 M3% = &H4000

230 M4% = SKETCH

240 CALL MOUSE (M1%,M2%,M3%,M4%)

Function 15 : SET MICKEY/PIXEL RATIO

Input                                   Output

M1% = 15

M3% = MICKEY/PIXEL RATIO (Horizontal)

M4% = MICKEY/PIXEL RATIO (Vertical)

Sets the mickey to pixel ratio for mouse motion (See the

section in this chapter, "The Mouse Unit of Distance: The

Mickey.") The horizontal and vertical ratios specify a number

of mickeys per 8 pixels.  The values must be in the range 1 to

32767.

The default  value for the horizontal ratio is 8 mickeys to 8

pixels.  The default value for the vertical ratio is 16

mickeys to 8 pixels.  In the default setting, it takes 6.4

inches of mouse of travel to move the cursor across the screen

horizontally, and 4.0 inches of travel to move it vertically.

Example

200 ' SET MICKEY/PIXEL RATIO AT 16 TO 8 AND 32 TO 8

210 M1% = 15

220 M3% = 16   ' Horizontal ratio

230 M4% = 32   ' Vertical ratio

240 CALL MOUSE (M1%,M2%,M3%M4%)

3. Systems interest

3.1 Driver operation

The Driver performs the Functions outlined in the previous

section. In addition on each hardware interrupt the following

sequence takes place:

- A count of the number of times the buttons are pressed and

released is maintained.

- The position of the cursor at the last button press is

maintained.

- The driver determines whether there has been any movement

of the Mouse since the last interrupt. The new position if

any is calulated and the cursor, if present, is moved to

the new location.

If the speed of the Mouse is determined to be above a

certain threshold then the speed of the cursor movement is

doubled.

***********************************************************************

######################################################################################