DISKID

-------------------------------------------------------------------

DISKETTE                     DESCRIPTION                       DATE

-------------------------------------------------------------------

PLOT			     VER 1.03			   17-12-83

-------------------------------------------------------------------

FILES           VER          DESCRIPTION

-------------------------------------------------------------------

PLOT    .EXE    1.03         Plotter Driver (Compiled BASIC)

PLOT    .BAS    1.03         Plotter Driver (BASIC source code)

MSBASIC .COM    5.28         Microsoft BASIC interpreter

DEMO80  .PLT                 Demo plot file for the ACT WRITER-80

DEMO81  .PLT                 Demo plot file for the ACT WRITER-81

DISKID                       Description of files - this file

README                       Instructions for use

COMMAND .COM    2.02         System file (See Port Configuration Note )

PRINTER .EXE	1.0          Utility to change the system printer port

-------------------------------------------------------------------

PRINTER PORT'S

The default port is parallel with the serial port set to work with both

the Writer 80 & 81 plotters. Use the PRINTER utility to change ports.

Parallel Port - This is the Default setting of the Operating System supplied

on this disk.

Serial Port - This is configured to 1200 Baud, 8 Data bits, 1 Stop Bit,

No Parity, RTS/CTS Protocol selected. These settings will work with either

the Writer 80 connected with a Writer 102 Serial interface cable or the

Writer 81 connected  with a Writer 103 Serial interfcae cable. To change from

the Parallel port (default) to the Serial port simply run "PRINTER SERIAL"

and to go back to the parallel port type "PRINTER PARALLEL".

********************************************************************************

Introduction for the BASIC PLOTTER DRIVER

This file contains details of the set of plotter driver routines supplied free

with this Plotter.These are intended as guidance for users and offer basic

facilities.

Written in MSBASIC (compiled) the PLOT program provides the user with high

level access to the plotter functions.In addition the un-compiled BASIC source

program is provided as an example, and demonstration, of the use of software

drivers for the ACT Plotters. Also enclosed are example programs DEMO80 and

DEMO81 which are example programs for use with this plotter driver.

Design of the Program

---------------------

In order to produce a language independent plotter driver the following

approach was taken;

As printing or plotting output from programs is quite slow in comparison to

the execution speed of the program all the commands are written to a sequential

disk file, instead of driving the list device directly, the file can then be

plotted at some other time.

By formally declaring the structure of the file, it would not matter which

language had produced it, as long as it could be read by the driver.

All filenames under MS-DOS consist of an 8 character filename and 3 character

extension, so as to differentiate between source code, (BAS .FOR .PAS),

object modules (.OBJ) and executable files (.EXE), the file extension .PLT

will be used to identify a plot file.

eg.   MAP021.PLT

Stucture of the System.

-----------------------

Two operations are required to produce a plotted output ; the generation

of the plot file and using PLOT.EXE to plot the file   ie.;

1.	A program or manual keying of commands produces file on disk

eg. ????????.PLT

2.	PLOT.EXE when called asks for the name of the file to plot

the named file is then output to the plotter.

Formal Definition of a Plot File.

--------------------------------

A plot file is held as a sequential disk file containing only printable ASCII

characters.  It can be produced by an applications program or by use of a

suitable text editor, and should conform to the following specifications :

i)    Only one statement is allowed on each line,

ii)   Blank lines are ignored.

iii)  Leading and  trailing spaces are ignored.

iv)   Input can be in either upper or lower case.

v)    Keywords and parameters must be separated from each other by at least

one space.

vi)   The space character is the only valid delimiter.

vii)  Lines must be terminated by a carriage return.

viii) If parameters are ommmited, they are assumed to be zero if numeric and

spaces if alphanumeric. This does not cause an error or warning to be

generated.

ix)   FATAL ERRORS are not recoverable and cause a return to MS-DOS. The

error message generated displays the type of error detected and the

line number at which it occured.

x)    The first command in a plot file must identify the type of plotter in

use. If ommitted this produces a FATAL ERROR.

xi)   The last command in a plot file is a DEVEND (or device end), to

de-allocate the plotter.  Failure to do so is not fatal, but will

generate a warning.

All commands must conform to the following syntax, (exmples are given of each

command).

Plot File Command Set.

----------------------

There are two classes of commands available to the programmer, configuration

and action commands. The command sets for each are described below :-

Configuration Commands.

-------------------------

WRITER-80

Declares the plotter type as the ACT Writer 80 (CX4800) and is used to set-up

all plotter dependent characteristics.

WRITER-81

As above, but for ACT Writer 81 (CX6000).

DEFINE LINES n   (ACT Writer 80 only)

Specifies the number of printing lines when the plotter is in PRINT mode.

When n = 0   48 lines

n = 1   infinite

Initially set to n = 0

DEFINE AREA n    (ACT Writer 81 only)

Specifies the effective plotting area.

When n = 0   area is 260 x 186mm

n = 1   area is 288 x 216mm

Initially set to n = 1

LINE SCALE n

Specifies the number of dots used to make dotted (or dashed) lines. The mark

to space ratio is 1:1, that is for each set of dots printed there is a gap

of the same length.

n is valid between 1 and 127

Initially set to n = 30

LINE TYPE n

Specifies the type of line drawn, solid or dotted.

n = 0  Solid line

n = 1  Dotted line

Initially set to n = 0

SET ORIGIN x y

Sets the origin for all LINE and MOVE operations etc... both x and y must be

within the plottable range.

Initially set to x = 0  y = 0

SET DIREC n

Specifies the print direction.

n = 0  Left to right

n = 1  Top to bottom

n = 2  Right to left

n = 3  Bottom to top

Initially set to n = 0

SET SIZE n

Specifies the size of printed characters.

n = 1  to 255

Initially set to n=4

SET MODE n

Sets the operational mode of the plotter.

n = 0  Graphics mode

n = 1  Print mode 1

n = 2  Print mode 2

Print mode 2 is only valid with ACT Writer 81.

Initially set to n = 0

STEP n      (ACT Writer 80 only)

Specifies the step size for the pen.

n = 0  0.05mm step

n = 1  0.10mm step

Initially set to  n = 1

DEVEND

Device end, terminates plotting of the file.

Action Commands.

------------------

ARC x y r q1 q2 d

Draws an arc of a circle based on x,y with radius r.

Centre         x,y  (-9999 to 9999)

Radius           r  (0 to 999)

Start angle     q1  (0 TO 360)

Finish angle    q2  (0 to 360)

Direction        d

d = 0  Anticlockwise

d = 1  Clockwise

AXIS p q r

Draws a single axis of a graph from the current pen position.

Axis    p  (0 specifies Y axis,

1 specifies X axis)

Step    q  (0 to 999)

Number  r  (1 to 255)

BAR x1 y1 x2 y2

Draws a bar or histogram defined by the diagonal (x1,y1) (x2,y2).

Position x,y = (-9999 to 9999)

BORDER c s m

Draws a border around the plottable area.

Colour    c = (0 to 3  ACT Writer 80

0 to 5  ACT Writer 81)

Step in   s = (0 to 9999)

Mode      m = 0 (border size 260 x 186)

1 (border size 288 x 216)

Mode only applicable to ACT Writer 81.

CIRCLE x y r

Draws a circle of radius r around the co-ordinates (x,y).

Centre  x,y = (-9999 to 9999)

Radius    r = (0 to 9999)

Leaves the pen at (x,y).

DIRECT string

Directly outputs the string to the plotter without any form of error checking.

EJECT

Sends a form feed to the plotter and redefines the plotting area on the next

page.(WRITER-80 ONLY).

FILL x1 y1 x2 y2 t p c

Fills the bar or histogram defined by the diagonal (x1,y1) (x2,y2) with a

coloured fill pattern.

Position   x,y = (-9999 to 9999)

Type         t = 0  horizontal shade

1  vertical shade

2  both of the above

Pitch        p = (1 to 9999)

Colour       c = (0 to 3 ACT Writer 80,

1 to 5 ACT Writer 81)

Leaves the pen at (x1,y1)

HOME

Moves the pen to the home position.

LINETO  x y

Draws a line to the absolute (x,y) co-ordinates.

Position x,y = (-9999 to 9999)

LINEBY x y

Draws a line of length (dx,dy), relative to the current pen position.

Displacement x,y = (-9999 to 9999)

MOVETO x y

Moves the pen to the absolute (x,y) co-ordinates.

Position x,y = (-9999 to 9999)

MOVEBY x y

Moves the pen by (dx,dy), relative to the current pen position.

Displacement x,y = (-9999 to 9999)

PEN n

Changes the pen colour.

Colour n = 0 to 3 (ACT Writer 80)

n = 0 to 5 (ACT Writer 81)

PRINT string

Prints the string from the current pen position. The results of this

facility depend on the direction and size of print selected.

Running the Demo Programs.

--------------------------

1.	Compiled version; from MS-DOS operating system prompt enter PLOT<CR>

you will then be prompted for a file, enter either DEMO80 or DEMO81

checking first that the plotter is ready.

2.	Interpreted version; from MS-DOS enter MSBASIC PLOT after loading

you will be promted for a file, enter DEMO80 or DEMO81 again checking

first that the plotter is ready. NOTE; that this Interpreted version

will be much slower than the compiled version.

END OF FILE

___________________________________________________________________________