CONTENTS

1.   Introduction

2.   Point 32 Components

3.   Using the Network

4.   Network Printers

5.   Physical Communication

6.   Network Software

7.   User Access to Network Files

8.   Applications in the Network Environment

8.1  File Protection and Sharing

8.2  Sharing Control

9.   Network Utilities

10.  Network Architecture

10.1 Physical and Data-link Layers

10.2 Network and Transport Layers

10.3 Session and Presentation Layers

.pa

Point 32 Overview

1.   Introduction

This  guide is an overview of the Point 32 Local  Area  Net#

work.   Readers should already be familiar with the principles of

computing, and ACT'S  microcomputers.

The    Network   consists   of   two  or   more    computers

physically  linked together.   Some advantages of using a network

are:

-    Sharing of resources (i.e.  file-space,  peripherals or

software),  allowing  these resources to be  used  more

efficiently.

-    Shared  files;  ensures consistency of data as only one

file has to be updated.  Also saves file-space.

-    Communication  between  users  can  streamline   office

procedures.

The   Point  32 Network is based on the Corvus  Omninet  and

Microsoft's  MS-NET.   OmniNet implements the hardware and  lower

level communication mechanism.  MS-NET implements  the  user  and

application   interface levels.  Together  they form an efficient

communications system for  interconnecting any Apricot computers.

Physically,   the   communication medium is  a  twisted-pair

trunk  cable.  The computers (with installed network hardware and

software), together with the communication medium,  make-up Point

32.   All the facilities provided  by the network are integral to

a standard version of the MS-DOS  Version  3.05 operating system.

(Point 32 cannot be  used  with earlier versions of MS-DOS).

It  is advisable for one person,   or a small team,   to  be

allocated the post  of  network  manager.   The  responsibilities

should   include installing  and  maintaining  network  software,

allocating   network resources,  and ensuring the  integrity  and

security of network data.  A Point 32 Manager's Guide is provided

for persons taking this role.

In   addition,   a  Workstation Users Guide is provided  for

users  and operators.

.pa

Point 32 Overview

2.   Point 32 Components.

Computers   in   the network are divided into two   classes.

These   are  Servers  which  provide  services or  resources   to

the  network,   and Workstations  (points  where  users can  gain

access to  the  resources provided by the servers).

Standard   models  within  the  Apricot  product  range  are

converted   into  servers  and workstations by the addition of  a

LAN Expansion Board and the appropriate network  software.    The

boards  are  supplied with a Tap Cable to link into  the  network

trunk.

A  Server  is a central computer with a hard disk  for  mass

storage;  it  is  based  on  an Apricot Xi  with  a  10  Megabyte

Winchester  disk.   Usually  it has no keyboard or  screen,   and

cannot  be used as a workstation.   A server may have  a  printer

connected  to  it,   and  the  servers'  disks  and  printer  are

available to the users of the network.  Each server may be fitted

with  one MSD 10Mb external Winchester disk,  giving a total hard

disk  capacity  of  20  Megabytes.   Point 32 may support  up  to

ten servers.

The   servers  use  a special control  program  that  allows

network  access to the file space,   controls user access to  the

server,  and  supervises  printing etc.    The hard  disk  stores

users' files,  files waiting to be printed (spooled prints),  and

the server software.  The floppy disk drive is used to initialize

the server,  and can be used for achiving files, and transferring

files from other systems.    Existing  Apricot XIs may be used as

servers, but must be fitted with a LAN board.

A   Workstation  is  a computer that is used to  access  the

server.  Any Apricot  capable  of running MS-DOS may be used as a

workstation (PC, Xi, Portable or F1 but no F1e).  As the stations

are standard Apricot machines,  they  have storage and processing

abilities  independent  of the  network,   and  thus   Point   32

implementation   uses  intelligent stations.    The  workstations

may  be used in local (normal stand-alone) mode,   or in  network

access mode.  A workstation is also used to issue commands to the

servers and display the servers' status.  Workstations must  have

a LAN board fitted and network  software  installed.  The network

supports up to 32 workstations, minus the number of servers.

.pa

Optionally,   Point  32 may support a number of Banks.   The

bank  is   a mass  storage  device that connects directly to  the

network,   and   is generally  used  for  archiving   files  from

the  servers  for  backup purposes.    It records data on a high-

speed tape cartridge,   and takes removable tape cartridge of 100

or 200 Megabytes capacity.

The  maximum number of devices that can be connected to  the

network  is 2,  of which up to ten may be servers.   The  minimum

configuration  is one  workstation and one  server.    Note  that

Point  32  is  not dependent on any one station  or  server,  and

network control is distributed.

Future   network devices will include larger  capacity  file

servers and gateway devices (for linking to the telephone network

- thus  providing the workstations access to  mainframes,   other

networks, public/provate database services, etc).

.pa

Point 32 Overview

3.   Using the Network.

This   release  of  Point  32  (V1.0)  allows  users  at   a

workstation to access files (both program and data) stored on one

or   more   servers.  Thus users may share common data files  and

load  programs  stored in the  server.   Note  that  applications

cannot  be  run in the servers;  they are dedicated to  providing

file   and  print  services.     All  applications  run  on   the

workstations.

The  resources  at the network servers are easily accessible

to the network users. A simple CONNECT command at the workstation

is  all that is required.  Workstations can also be used in local

mode without any loss of efficiency or functionality.

The  directories  and files at the server are made to appear

as   local  drives  or  pathnames   at   the   workstation;   the

Point  32  software redirects  all  commands  for remote (server)

files   and   peripherals across  the  network.    The  user  can

perform most of the  MS-DOS  file commands   (such  as  COPY  and

DIR)  on  these   directories,   load applications into the local

station,   and generally use these files as if they were local to

the  station.    The server directories and  files must  be  made

available to users by commands to the server, and these resources

may   be  password protected.    See the  section   on   Resource

Managment for further details.

.pa

Point 32 Overview

4.   Network Printers.

Each  server  can  support on printer that is  available  to

network  users.  Users  access the printer via a  simple  CONNECT

command,  and thereafter the  printer appears as if it were local

to  the workstation.    A spool queue  in  the server is used  to

hold print files until  they  can  be printed,  and the queue  is

handled is a 'first-in, first-out' fashion.

Output  can  also  be directed to a local printer (connected

to  the workstation) of one is present.

.pa

Point 32 Overview

5.   Physical Communication

In   Point  32,   the  physical communication  mechanism  is

implemented by the  Corvus OmniNet.    The Corvus system is based

upon an inexpensive twisted  pair trunk cable as the transmission

medium,   using  a  common bus  structure.    This cable  or  bus

forms  the  backbone  of the network.  Linking into  the  network

requires a junction box (referred to as a Tap box) to provide the

means on connecting a computer to the trunk.   The Tap  Boxes are

passive devices and of a comparitively  simple  design.  Figure 1

shows a simple network configuration.

(as per h/ware installation guide)

Figure 1. Basic Network Configuration

Using  passive Tap Boxes and unshielded twisted pair  cable,

the maximum length  of  the network can be up to 2000 feet (hence

the  term  Local Area Network).

Active   junction boxes enable the network to be extended to

4000  feet.   They will also allow branches to be made  from  the

main  network  trunk,  which is a feature not available using the

passive devices.

The   LAN board is installed within the computers by  simply

plugging it into an available Expansion Slot.    Connecting a new

station  or server into the network is a simple procedure and  is

achieved by plugging one end  of the Tap Cable into the LAN board

and the other end into a  Tap Box.

All   workstations  and servers on the network have  to   be

assigned a unique  physical address to allow the network software

to  identify the source and destination of data transferred  over

the network.

.pa

This  address  is  set  up via a series of microswitches  on

the  LAN boards.    This addressing technique allows stations and

servers to  be physically moved without effecting the network.

The  LAN  boards provide an RS422 interface to  the  network

bus,  and  a DMA channel for use in passing messages to and  from

its   host  processor.   The active junction boxes  are  standard

RS422 repeaters.

The   protocol   used is CSMA/CA with a transfer rate  of  1

Megabit  per second.  See Section 10 for further details.

.pa

Point 32 Overview

6.   Network Software.

The   communications  features provided by the  network  are

controlled by the  networking  software. This software has to  be

installed  and configured  before the network can be operational.

Details of the installation of the network software is covered in

a  separate document, but the following sections give an overview

of its structure and capabilities.

The   system software for both the servers abd  workstations

is provided by MS-DOS version 3.05 and MS-NET Version 1.0.

MS-DOS  3.05  is an upgraded version of  2.11.  It  includes

embedded  basic network access functions and implements file  and

record locking.  Version 3.05 is also a standard operating system

for  non-networked environments.

MS-NET  is  only  loaded  in  networked   environments;   it

intercepts  all MS-DOS  calls  and  routes these calls across the

network  as  and  when necessary.

.pa

Point 32 Overview

7.   User View of Network Files

Resource   management   and  file control in  Point  32  are

implemented  by  MS-NET  and MS-DOS.    The  two  major  software

components of MS-NET  are server and redirector.   Server runs as

an  application  in  all servers,  and  provides file  and  print

services  to  the network.    Redirector  is installed  in  every

workstation,   and performs transparent  redirection of file  and

print requests from the workstation to the network.

Directories  at  a server are 'offered' to the network to be

used  by remote  users.    The  directory and the file  structure

beneath it  are accessible to workstations.    Unless so offered,

directories  and  their  substructure are not  available  to  the

users.  When  offered,  directories may have passwords and access

controls  placed   on  them.  These  are   checked   whenever  an

offered directory is  accessed  by  a remote user.

As  an example,  suppose the following file structure exists

on a server:-

A:  = root

-------------------------------------------

\accounts            \payroll             \customer

----------------                           --------------

\invoice       \ledgers      file1         \credit     address

The root directory (A:) can be offered on the network with a

password that  is  only  known to the network manager.   Thus the

manager has access to all files on that server. Another offer can

be made for the directory\accounts,  again with a password. Users

connecting to\accounts are able to access this directory, and all

directories and files  in its substructure,   but are not able to

access (and may not even be aware of) the file structures\payroll

and\customer.

.pa

Offers  also specify the access mode allowed for a directory

or file structure.   Access mode may be 'read only' or  'read and

write'.

All  offer  commands at a server may use an  alias  for  the

directory,  and  different  passwords  and access  modes  may  be

attached to each alias.

Thus  the directory\payroll may be given one alias with read

and  write access  and  associated password,   and another  alias

with  read  only access and a different password.

Each server is assigned a name. This name is used in connect

commands  at  workstations  to  identify the file as remote   and

the  required server.    It is preceeded by two backsplashes.   A

pathname to a remote file has the form:-

\\server\directory\[directory\...]

If the example file system resides on the server named BLUE,

and  \customer is offered without an alias, a connect command  to

\customer has the form:-

\\BLUE\CUSTOMER\

The  connect  command will include a password,   if  one  is

required, and all subsequent requests using this pathname will be

redirected  across the network.  The connect command may also tie

the pathname to an used local drive letter (e.g.  C:  to F:). All

references to the drive are then redirected to the pathname.

Pathnames will also be extended in further releases of Point

32 to include a network name, thus permitting internetworking.

.pa

Point 32 Overview

8.   Applications in the Network Environment.

Applications software and data may be stored on the  server,

and may make use of the disk and printer resources at the server;

no applications may run in the server.

Any applications written for MS-DOS 1.0,   1.25,   2.0,  and

2.11 may run on Point 32, with the following caveats:

1)   The  application  uses  the  MS-DOS  environment,  and

particularly disk accesses are done via MS-DOS.

2)   Once the application has been loaded in the workstation

it may access the printer,  screen,  keyboard, etc. via

the  BIOS,  but I/O via the BIOS cannot be  redirected.

Print  output must be through MS-DOS to use the  server

printers.

3)   Applications  stored  on  the  server  and  used  from

multiple workstations must not be self-modifying.   All

code, overlay, and message files must be read only (and

marked  as such).  Applications' remote data may not be

used  concurrently without amending the application  to

include record and file locking.

Existing applications cannot interfere with each other,  for

example  if a  user is editing a file with WordStar then a second

user  attempting to open that file would be rejected.

In  order  to  utilise  the full  advantages  of  Point  32,

applications  should   be  upgraded to include  file  and  record

locking.    This can  be easily done using an extension in MS-DOS

3.05.

.pa

Point 32 Overview

8.1  File Protection and Sharing.

In  the  current  release  of Point  32,  all  resources  on

stations are assumed to be local to that station,  and cannot  be

accessed   from elsewhere in the network.   This ensures somplete

privacy  of  files on a station,   and  that  no-one is  able  to

'steal'  a local printer in  the middle of a  print  job.    Only

resources on file servers are shareable, and can be accessed from

more than one station at a time.

MS-DOS  3.05  implements  the functions of 'sharing control'

(file locking) and 'locking control' (record and block locking).

These  are essential features for a networking  environment,

and   are  required  for avoiding  inconsistent  data,  and  file

contentions.

Access  modes  offered  with  a  file  structure  have  been

discussed   above.  When a program opens a file in the structure,

it  must establish a 'use mode'  The use mode may be  Input-only,

Output-only,   or Input/output,  and must conform with the access

mode of the file itself.   use mode is a property of the  program

opening the file.

8.2  Sharing Control.

Once  access has been granted,  the sharing of files must be

controlled. Access  control limits what a user can do independent

of  other   users:  sharing  control limits what a  user  can  do

because  of  what other users are doing.    The sharing  mode  is

established  when  the file is  opened.  There are  four  sharing

modes;  these  are 'deny all others' (DRW),  'deny all   readers'

(DR),   'deny other writers' (DW),   and 'deny all' (ALL). Should

the  requested sharing mode not be allowed because it   conflicts

with   a  sharing mode used by another user that already has  the

file open, the file will not be opened.

The  four  sharing modes,in conjunction with the  three  use

modes,  provide  a wide range of contention resolutions.  Some of

the possibilities are listed in Table 1.

.pa

Access     Use   Sharing          Description

R/W        I/O     DRW        Exclusive, one reader/writer

R           I      DRW        Exclusive, one reader

R           0      DRW        Exclusive, one writer

R/W         0       DW        Shared, one writer, may readers

R/W        I/O      DW        Shared, one reader, many writers

R           I      ALL        Shared, many readers

R           O      ALL        Shared, many writers

R/W        I/O     ALL        Shared, may readers/writers

Table 1.  Some File Access Options.

In  order  to take advantage of the many options  available,

applications  should   ideally  be implemented for  a  networking

environment  to   avoid contention problems between  users  while

maximising file availability.

Access control and sharing control are on a per-file  basis.

Within  a file users can also control the use of the contents  of

the file by use of  'block locking'.    This permits any block of

characters   within  a file to be locked,   preventing access  to

other users while the lock is in effect.

.pa

Point 32 Overview

9.   Network Utilities.

Utilities  will  be  supplied for assisting  in  configuring

Point 32,  for example  generating  address files,  and offer and

connect  commands. Archive and restore utilities for use with the

bank are also provided.

.pa

Point 32 Overview

10.  Network Architecture.

Point   32  implements a network architecture based  on  the

ISO  7-layer model  for,   Open  Systems  Interconnection.   Each

layer   uses   the  facilities made available by the  nest  lower

layer  to provide  services to the next upper layer.   Thus  each

layer  is  shielded  from the details of  how  the  lower  layers

implement these services.   Figure 2 shows the arrangement pf the

seven layers.

Name                  Layer

-----------------

Application               7

-----------------

Presentation               6

-----------------

Session                5

-----------------

Transport                 4

-----------------

Network                  3

-----------------

Data Link                 2

-----------------

Physical                  1

-----------------

Figure 2. The ISO Model for Open Systems Interconnection.

The layered approach allows any layer to be replaced without

affecting  other  layers.    For example,   application  software

written  for  MS-NET will  run  on  any  implementation  of   MS-

NET,  irrespective  of  the implementation  of  the lower layers.

However,   note that  the  same implementation of all layers must

be present in all machines within  a single network.

10.1 Physical and Data Link Layers.

Layers   1  and  2,   being the physical connections and the

data  link protocol,   are  supported by the Corvus' OmniNet chip

set (located  on the  LAN  board).   The physical  implementation

has been  discussed  in section 4.

The Data-link layer is responsible for accepting information

from  the Network   layer   and  assembling  it  into  a   format

suitable    for transmission,   and  accepting received data  and

passing it up  to  the Network layer.

The   protocol  used in Point 32 is Carrier-Sense  Multiple-

Access  with Collision Avoidance (CSMA/CA).   Briefly, this means

that  all  stations and  servers broadcast on the  network,   and

each  listens  only  for  its own address.    None  can  start  a

transmission while one is in  progress (carrier-sensing).

If   two  start a transmission at the same time,   both halt

and   retry  after  a  random   period   (collision   avoidance).

The   protocol  is responsible  for  checking  the  integrity  of

all   messages  which  it transmits  or receives.    The transfer

rate on the cable is 1  Megabit per  second,    with  about   20%

overhead  for  error  detection   and correction.

10.2 Network and Transport Layers.

The  network  and  transport layers of Point  32  have  been

written by ACT.  The  network layer in the OSI model is concerned

with  routeing through the network,   and its significance  in  a

CSMA/CA  network is negligible.  The  transport layer  implements

the  services required by MS-NET  using the  OmniNet  facilities.

These   services   include  virtual   circuit  establishment  and

management,  and a datagram service: in short, these are means by

which end-to-end communication is established, regardless of  the

lower  layers.    Part of the Transport layer's function is   the

management   of   transmission  and reception  buffers  via   the

transport protocol credit packets.    (A credit packet is sent to

the other  side when buffer space is available to receive it).

Note   that  no  standards have yet been  defined  for  the

Transport layer, and other vendors implement it other variations.

Thus  Point 32  will not  be  compatible  with  any given network

even  if it  is  based  on OmniNet and  MS-NET.    However,   ACT

will supply Transport software for other vendors equipment.

10.3 Session and Presentation Layers.

The  Session and Presentation layers are implemented in  MS-

NET and  MS-DOS.   Together they provide the users' interface  to

the  network,  and   manage  the intra-network  dialogues  in  an

orderly   manner.   The  Session  layer   provides  the  control,

delimitation  and synchronisation of data exchange,   and   helps

manage  file access control  by  performing  the  permission  and

password functions.

The Presentation layer prepares the data for the application

layer;  this involves the transformation of information so   that

the  differences  between systems on the network are  eliminated.

In  effect,  the   Presentation  layer  hides  the  network  from

applications, so that applications may use local and remote files

without special  network operations.

##########