SCRNCOPY

~~~~~~~~

SCRNCOPY is a utility to enable the apricot range of computers

to be able to copy both text and graphics from the screen to the

printer.

This utility is accessed when installed by pressing the <CONTROL>

and <PRINT (F6)> keys together. It is also available using the

LCOPY statement within GWBASIC.

If you wish to use this facility within your own programs, details

of how to access this routine is provided in the Apricot "BIOS 3

SOFTWARE TECHNICAL REFERENCE MANUAL" Section 6, function 91.

This utility is installed as follows:-

SCRNCOPY <printer.prd> [/Wn /A /B /F]

where:-

/Wn     sets paper width (inches). eg to set

the page width to 12.5 inches use /W12.5.

/W      sets page width to 13.5 inches.

The default page width is 8 inches.

/A      prints a 'FORM-FEED' character after

printing.

/B      prints a 'FORM-FEED' character before

printing.

/F      Do not filter non-printable characters

before printing text screen.

/Q      Emphasis on quality of reproduction,

default is to maintain aspect ratio.

/R      Print in reverse video.

Examples:-

SCRNCOPY WRITER22.PRD /W7.5 /B

Installs the Writer 22 for use with 7.5 inch paper and to

preceed each screen copy (text and graphics) with a form-

feed.

SCRNCOPY WRITER12.PRD /W14 /A /B

Installs the Writer 12 for use with 14 inch paper and to

print a form-feed character both before and after each

screen copy (text and graphics).