November 1984

Control Device Documentation

******************************

1    INTRODUCTION

The Control Device driver is a generic interface

to I/O functions across the whole Apricot range.

It provides the applications programmer with a

very easy method of writing I/O routines. Because

of the generic and standardised nature of the

Control Device, applications using it are

portable across the Apricot range.

All I/O accesses should be made via the Control

Device instead of directly to the hardware as

this permits portability of the applications

across the Apricot range.

Calls and interrupts to the Control Device are

validated and passed to the relevant device

driver for execution. It thus provides

applications programmers with a simplified method

of using the device-driver routines in   BIOS  .

For example, the same routine can be used from

within a program to access all devices including

the floppy disks, serial I/O port etc. Note that

MS-DOS calls to the BIOS are also routed via the

Control Device.

The Control Device is currently implemented in

the F1 and Portable models, and a fully

functional version for the Apricot PC and Xi

machines will be available in the near future.

Currently a limited form of the Control Device is

available on the Apricot PC and Xi range. This is

a subset of the Control Device described here,

and functions not supported by PC and Xi are so

marked.

1.2  Using the Control Device

The Control Device allows for the setting and

interrogating of various BIOS driver parameters

by application programs, and of the ROM/RAM BIOS

interface. For example, the Serial port driver

can be initialized, and Baud rates etc. can be

set and reset as required.

Note that initializing a driver (COM = 0000)

causes that driver to be re-initialized to the

boot state as described in the previous chapter.

For example, initializing the screen driver

causes the screen to be cleared and all

attributes reset.

When using the Contol Device and BIOS to access

disks, care should be taken to avoid corrupting

disks.

Two methods of calling the Control Device are

provided: via an interrupt, suitable for assembly-

level programming, or by a CALL IO used from a

high-level language (e.g. MS-Basic).

To access the Control Device via an interrupt,

use INTERRUPT 0FCH.

Parameters are passed and returned via registers:

ENTRY:

BX = Device number        (DEV)

CX = Command              (COM)

DX = Data                 (DAT)

Optionally DX:SI is a double word pointer to a

data table. This is dependent on Device and

Command type. (DX = Segment, SI = offset).

EXIT:

AX = Returned data/status (RET)

Optionally, additional data may be returned in

the data table (see above).

All registers except AX are preserved.

DEV, COM, DAT and RET are the equivalent words

when CALLing the routine, and the registers are

referred to by those names in the text for

particular commands.

To access the Control Device via a call, use FAR

CALL TO 0600H.

Parameters are passed via the stack, and four

stack entries are required. These are four word-

length pointers to parameters in the current Data

Segment.

First Parameter pushed:

Word Pointer to Device Number (DEV)

Second Parameter pushed:

Word Pointer to Command       (COM)

Third Parameter pushed:

Word Pointer to Data out      (DAT)

Fourth Parameter pushed:

Word pointer to Data/Status return

(RET)

Optionally the third and fourth parameters form a

double-word pointer to a data table. In this case

the third parameter is the segment address and

the fourth is the offset. The data table may be

used for data out and return. All registers are

preserved.

1.3  Device Numbers

Valid device numbers (DEV) are as follows:-

ASCII   HEX     Device

1       31      Screen Driver

2       32      Keyboard Driver

3       33      MicroScreen Driver

4       34      Serial I/O Driver

5       35      Parallel I/O Driver

6       36      Mouse Driver

7       37      Clock Driver

8       38      Sound Generator /

Key click / Bell

9       39      Floppy Disk Drivers

@       40      Winchester Disk Drivers

A       41      Modem Board Drivers

B       42      Cache / Graphics /

IBM configuration

1.4  Returned Status Codes

These codes are returned by any device when an

"Initialize Device" call (command '0'), or I/O

command is issued:

0000H   -  Device present and on-

line, status OK

8000H   -  Write Protect

8001H   -  Unknown Unit

8002H   -  Not Ready

8003H   -  Unknown Command

8004H   -  CRC Error

8005H   -  Bad Request Length

8006H   -  Seek Error

8007H   -  Unknown Media

8008H   -  Sector Not Found

8009H   -  Printer out of Paper

800AH   -  Write Fault

800BH   -  Read Fault

800CH   -  General Failure

FFFFH   -  Device Error (also invalid

device/command)

(FFFFH is equivalent to -1 in Basic)

Note: These codes may be returned in place of any

codes that are returned in response to specific

commands for devices listed below. Some status

returns set the 0200H bit to indicate device

busy.

Calls to non-existent device drivers are trapped

by the Control Device, and return Device Error

(FFFFH). No other action is taken.

2    Device-Specific Commands

The following sub-sections list all commands

available for each device type.

Symbols in the left-hand column indicate where

commands are not supported by particular models.

These are as follows:

# - Not currently implemented on Apricot pc, xi.

% - Not implemented on Apricot Portable.

$ - Not implemented on Apricot F1.

Model-type can be determined by reading the

one-byte value at 401H. (i.e. the

Pointers/Constants area).

0 = Apricot pc, xi

1 = Apricot Portable

2 = Apricot F1.

2.1  Screen Driver DEV = 0031H

COM = 0000H    Initialise Driver

DAT = not used

RET = 0000H, ok

COM = 0001H    Get/Set Text/Graphics Display Mode

DAT = 0000H, Set Text Mode

DAT = 0001H, Set Graphics mode

RET = setting (current if DAT invalid)

COM = 0002H   Get/Set Display On

DAT = 0000H, Display on

DAT = 0001H, Display off

RET = setting (current if DAT invalid)

COM = 0003H   Copy virtual screen image to

real screen (if option installed)

DAT = 0, Update Colour screen (F1 & Portable)

DAT = 1, Update LCD screen (Portable)

RET = 0000H

COM = 0004H   Print single character

DAT = Character to print (lower byte)

RET = 0000H

Note: the use of INT 0F1H is somewhat faster

COM = 0005H    Print string

DEV and RET (DX:SI) is a pointer to:

WORD   number of characters to output

DWORD  pointer to string

RET = 0000H (returned)

2.2   Keyboard Driver, DEV = 0032H

COM = 0000H    Initialise Driver

DAT = not used

RET = 0000H, ok

RET = 0001H, keyboard missing

COM = 0001H    Get/Set Auto-repeat

DAT = 0000H - Disable auto-repeat

DAT = 0001H - Enable auto-repeat

RET = setting (current if DAT invalid)

COM = 0002H    Get/set Auto-repeat lead-in

delay (20ms intervals)

DAT = 0001H - 00FFH (times 20ms)

RET = setting (current if DAT invalid)

COM = 0003H    Get/set Auto-repeat rate

(20ms intervals)

DAT = 0001H - 00FFH (times 20ms)

RET = setting (current if DAT invalid)

COM = 0004H    Get/Set keyboard fall-through mode

DAT = 0000H - keys interpreted by BIOS

DAT = 0001H = Raw up/down codes passed to DOS

RET = setting (current if DAT invalid)

Note: the infra-red keyboard only produces

downcodes, and no code is generated for SHIFT or

CONTROL keys. Downcodes are Apricot compatible.

COM = 0005H    Flush keyboard input buffer

DAT = not used

RET = 0000H

COM = 0006H    Reset Keyboard control/shift

status and clear downcode buffer.

DAT = not used

RET = 0000H

COM = 0007H    Get/Set HELP key detection in

fall-through mode.

DAT = 0000H - HELP key terminates fall

through mode.

DAT = 0001H - HELP key ignored.

RET = setting (current if DAT invalid)

COM = 0008H    Place data in keyboard buffer

DAT = ASCII data (lower byte)

RET = 0000H - ok

RET = FFFFH - no room in buffer (bell

sounds)

COM = 0009H    Sound the bell

DAT = not used

RET = 0000H

COM = 000AH    Return number of keys in

keyboard buffer

DAT = not used

RET = number of keys in buffer (0 = none)

COM = 000BH    Get Key from Keyboard Buffer

(waits if none available).

DAT = not used

RET = ASCII key data (lower byte).

COM = 000CH    Return next character in queue but

do not extract it (look-ahead).

DAT = not used

RET = 00XXH - next character in buffer

RET = 0FFFFH if no characters.

COM = 000DH    Place string in keyboard

buffer

DAT  (DX:SI) is pointer to:

WORD Byte count (80 max. not

checked)

DWORD Pointer to string

RET = 0 if string fitted

COM = 000EH    Get string from Keyboard

buffer

DAT  (DX:SI) is pointer to:

WORD  Byte count

DWORD Pointer to string space

RET = 0

COM = 000FH    Get/Set Keyboard status

DAT: DH contains the AND mask and DL

the OR mask for the status byte.

Status byte is returned in AL.

0 = off, 1 = active.

bit 0 = Ignore HELP key in raw mode

bit 1 = STOP mode on

bit 2 = CALCULATOR mode on

bit 3 = SHIFT LOCK on

bit 4 = raw mode on (raw downcodes sent

direct to DOS)

bit 5 = VOICE LED on

bit 6 = KEYBOARD LOCK on

bit 7 = CAPS LOCK on

RET = Status byte

For example, to get the current status without

changing any, send DAT = 0FF00h (status is AND'ed

with 0FFh, then OR'ed with 00h - end result is no

change). To set the STOP mode on send DAT=0FD02h,

etc.

2.3   MicroScreen Driver, DEV = 0033H

COM = 0000H    Initialise Driver

DAT = not used

RET = 0

COM = 0001H    Print character (lower 8 bits

of data word)

DAT = character to print (lower byte, DL)

RET = 0.

COM = 0002H    Send character straight to

MicroScreen/Keyboard as raw,

non-interpreted, output.

DAT = raw data to keyboard unit

RET = 0.

2.4  Serial Port Driver, DEV = 0034H

COM = 0000H    Initialise Driver

DAT = not used

RET = 0.

COM = 0001H    Transmit Character (lower 8

bits of data word)

DAT = Character to transmit (lower byte)

RET = 00000H - ok

RET = 0FFFFH - failed.

COM = 0002H    Receive character

DAT = not used

RET = 0FFFFH - no character available

RET = 000XXH - received character

COM = 0003H    Update SIO settings as set by

calls 0004H to 0009H, 0018h

to 0019h.

DAT = not used

RET = 0.

COM = 0004H    Get/Set Transmit Baud rate

DAT = 1 to 15, baud rate setting

(See table at end of section)

RET = setting (current if DAT invalid)

COM = 0005H    Get/Set Receive Baud rate

DAT = 1 to 15, baud rate setting

(See table at end of section)

RET = setting (current if DAT invalid)

COM = 0006H    Get/Set Transmit bits per

character

DAT = 5 to 8, bits per character

RET = setting (current if DAT invalid)

COM = 0007H    Get/Set Receive bits per character

DAT = 5 to 8, bits per character

RET = setting (current if DAT invalid)

COM = 0008H    Get/Set number of stop bits

per character

DAT = 1 to 3, stop bit code (1 = 1 stop

bit, 2 = 1.5, 3 = 2 stop bits).

RET = setting (current if DAT invalid)

COM = 0009H    Get/Set Parity Type

DAT = 0 to 2, parity type code

(data: 0=none, 1=odd, 2=even).

RET = setting (current if DAT invalid)

COM = 000AH    Get/Set Transmit XON/XOFF protocol

DAT = 0, disabled

DAT = 1, enabled

RET = setting (current if DAT invalid)

COM = 000BH    Get/Set Receive XON/XOFF protocol

DAT = 0, disabled

DAT = 1, enabled

RET = setting (current if DAT invalid)

COM = 000CH    Set/Reset/Get RTS line

DAT = 0, reset

DAT = 1, set

RET = setting (current if DAT invalid)

COM = 000DH    Set/Reset/Get DTR line

DAT = 0, reset

DAT = 1, set

RET = setting (current if DAT invalid)

COM = 000EH    Set/Reset/Get Transmit enable

DAT = 0, disable

DAT = 1, enable

RET = setting (current if DAT invalid)

COM = 000FH     Get/Set Keyboard Status

DAT = 0, disable

DAT = 1, enable

RET = setting (current if DAT invalid)

COM = 0010H    Get CTS line status

DAT = not used

RET = 0, reset

RET = 1, set

COM = 0011H    Get DCD line status

DAT = not used

RET = 0, reset

RET = 1, set

COM = 0012H    Get DSR line status

DAT = not used

RET = 0, reset

RET = 1, set

COM = 0013H    Set/Reset nulls after CR

(carriage return)

DAT = 0000H to 00FFH nulls (0 = disabled)

RET = setting (current if DAT invalid)

COM = 0014H    Set/Reset nulls after FF

(form feed)

DAT = 0000H to 00FFH nulls (0 = disabled)

Note: ten times the number of

nulls will be sent.

RET = setting (current if DAT invalid)

COM = 0015H    Set/Reset auto LF after CR

DAT = 0, disable

DAT = 1, enable

RET = setting (current if DAT invalid)

COM = 0016H    Enable serial mouse

(Serial receive data generates an

INT 0FAH with the data in AL).

DAT = not used

RET = 0.

COM = 0017H    Disable Serial Mouse.

DAT = not used

RET = 0.

COM = 0018H    Set/Reset RTS/CTS protocol

DAT = 0, disable

DAT = 1, enable

DAT = 2, Auto-enables mode

RET = setting (current if DAT invalid)

COM = 0019H    Set/reset DTR/DSR protocol

DAT = 0, disable

DAT = 1, enable

DAT = 2, Auto-enables mode

RET = setting (current if DAT invalid)

COM = 001AH    Set/Reset External SIO control

$

DAT = 0, SIO controlled by BIOS

DAT = 1, SIO RS232 channel interrupts

vectored via interrupt 0F0H.

RET: Low byte (AL) = Base SIO port address

Order: [base+0] = Ch A data

[base+2] = Ch A status

[base+4] = Ch B data

[base+6] = Ch B status

High byte (AH), 0 = RS232 on channel A,

1 = channel B.

COM = 001BH    Return next received character but

do not remove from queue

(look-ahead).

DAT = not used

RET = 000XXH, received character (0FFFFH if

none available).

COM = 001CH    Flush Transmit Buffer

DAT = not used

RET = 0.

COM = 001DH    Flush Receive Buffer

DAT = not used

RET = 0.

COM = 001EH    Get Receive Character status

(and reset error bits).

DAT = not used

RET = Status in lower byte as for SIO Read

Register 1, i.e.

Bit 6 - CRC / framing error

Bit 5 - Rx overrun error

Bit 4 - Parity error

(See also SIO description)

COM = 001FH    Transmit string

DAT (DX:SI) is a pointer to:

WORD  Byte count

DWORD String pointer

RET = 0 if string transmitted, or

RET = error code.

Note: Byte Count field is updated with

number of bytes transmitted

COM = 0020H    Receive string

DAT (DX:SI) is a pointer to:

WORD  Byte count

DWORD String space pointer

RET = 0

COM = 0021H    Get/Set receive queue length

(512 bytes max)

DAT = 1 to 512, queue length

RET = setting (current if DAT invalid)

COM = 0022H   Get/Set transmit queue length

(512 bytes max)

DAT = 1 to 512, queue length

RET = setting (current if DAT invalid)

TABLE 1

Values for Baud rates are as follows:-

1     -     50 baud

2     -     75 baud

3     -     110 baud

4     -     134.5 baud

5     -     150 baud

6     -     300 baud

7     -     600 baud

8     -     1200 baud

9     -     1800 baud

10    -     2400 baud

11    -     3600 baud

12    -     4800 baud

13    -     7200 baud

14    -     9600 baud

15    -     19200 baud

Values for stop bits are as follows:-

1     -     one

2     -     one and a half

3     -     two

Values for Parity type are:-

0     -     no parity

1     -     odd parity

2     -     even parity

2.5  Parallel I/O driver, DEV = 0035H

COM = 0000H    Initialise Driver

DAT = not used

RET = 0

COM = 0001H    Return number of bytes free in

printer output buffer (if less

than 3 regard as full)

DAT = not used

RET = free bytes.

COM = 0002H    Get/Set FAULT line status detect

$

DAT = 0, disable

DAT = 1, enable

RET = setting (current if DAT invalid)

COM = 0003H    Get/Set SELECT line status detect

$

DAT = 0, disable

DAT = 1 - enable

RET = setting (current if DAT invalid)

COM = 0004H    Get/Set PAPER ERROR line status

detect

$

DAT = 0, disable

DAT = 1, enable

RET = setting (current if DAT invalid)

COM = 0005H    Get/Set Auto LF after CR enable

DAT = 0, disable

DAT = 1, enable

RET  = setting (current if DAT invalid)

COM = 0006H    Get/Set Serial/Parallel PRN:

device output

DAT = 0, Parallel PRN:

DAT = 1, Serial PRN:

RET = setting (current if DAT invalid)

COM = 0007H    Print character on device set by

call 0006H (Serial or Parallel)

DAT = Character in lower byte (DL)

RET = 0, ok, or

RET = error code

COM = 0008H    Flush printer output buffer

DAT = not used

RET = 0.

COM = 0009H    Transmit string (serial or parallel)

DAT and RET (DX:SI) is a pointer to:

WORD  byte count

DWORD Pointer to string

RET = 0 if string transmitted, or

RET = error code.

COM = 000AH    Return output status

DAT = not used

RET = 0 ok, or

RET = Printer busy, or no paper code.

2.6  Mouse driver, DEV = 0036H

Not implemented

2.7  Clock Driver, DEV = 0037H

COM = 0000H    Initialise Clock Driver and resets

time to 0:00:00 1-1-80

DAT = not used

RET = 0

COM = 0001H    Set Date/Time

DAT and RET (DX:SI) points to data table:

WORD  Days Since 1-1-80

BYTE  Minutes

BYTE  Hours

BYTE  Hundredths Of Seconds

BYTE  Seconds

RET = 0.

COM = 0002H    Get Date/Time

DAT and RET (DX:SI) points to 6-BYTE data

space for above table.

RET = 0.

2.8   Sound Generator / Key click / bell,

DEV = 0038H

COM = 0000H    Initialise Sound Driver

DAT = not used

RET = 0.

COM = 0001H    Get/Set Key feedback click volume

DAT = 0 to 15 (0 = max, 15 = min)

RET = setting (current if DAT invalid)

COM = 0002H    Get/Set bell volume

DAT = 0 to 15 (0 = max, 15 = min)

RET = setting (current if DAT invalid)

COM = 0003H    Set channel 2 volume

$

DAT = 0 to 15 (0 = max, 15 = min)

RET = setting (current if DAT invalid)

COM = 0004H    Set channel 3 volume

$

DAT = 0 to 15 (0 = max, 15 = min)

RET = setting (current if DAT invalid)

COM = 0005H    Set channel 2 frequency

$

DAT = 0 to 1023 (0 = max, 1023 = min)

RET = setting (current if DAT invalid)

COM = 0006H    Set channel 3 frequency

$

DAT = 0 to 1023 (0 = max, 1023 = min)

RET = setting (current if DAT invalid)

COM = 0007H    Set bell frequency

DAT = 0 to 1023 (0 = max, 1023 = min)

RET = setting (current if DAT invalid)

COM = 0008H    Set bell duration

DAT = 1 to 255 (times 20ms)

RET = setting (current if DAT invalid)

COM = 0009H    Sound Bell

DAT = not used

RET = 0

COM = 000AH    Sound Click

DAT = not used

RET = 0

2.9  Floppy Disk drivers, DEV = 0039H

COM = 0000H    Initialise Drivers

DAT = not used

RET = 0

COM = 0001H    Set Drive number for format

DAT = 0 or 1 (floppy 0 & 1)

RET = setting or 0FFFFh if invalid

COM = 0002H    Set Segment address of Track image

for Format

DAT = segment address of image

RET = 0

COM = 0003H    Set Offset address of Track image

for format

DAT = offset address of image

RET = 0

COM = 0004H    Format Disk (floppy only)

DAT = 0   70 track SS

DAT = 1   80 track SS

DAT = 2   80 track DS

RET = 0, ok, or Error status

COM = 0005H   Return status of requested disk

DAT = 0 or 1 (drive 0 or 1)

RET = status as WD279X type 1 status, or

RET = 0FFFFH if invalid.

COM = 0006H    Set disk status to swapped

DAT = 0 or 1 (drive 0 or 1)

RET = 0, or 0FFFFH if invalid.

COM = 0007H    Return disk type

DAT = drive number (0 or 1)

RET = 0       70 track SS floppy

1       80 track DS floppy

2       80 track DS floppy

80H     No drive fitted

COM = 0008H    Return drive type

DAT = drive number (0 or 1)

RET = 0   70 track SS floppy

1   80 track SS floppy

2   80 track DS floppy

80H No drive fitted

COM = 0009H    Check For Disk Swapped

DAT = drive number (0 or 1)

RET = 0   Disk not swapped

1   Disk swapped

2   No Disk

Note: If RET = 1 is returned, a COM = 6 must

be done, else swapped status will be lost.

COM = 000AH    Get BPB (this should be called

whenever a disk has been swapped).

DAT and RET (DX:SI) points to a data table:

WORD  Drive (0 or 1)

DWORD Pointer to 512 Byte scratch space

RET = return status, and pointer to loaded

BPB in DWORD.

COM = 000BH    Read/Write/Verify/Write with

Verify.

DAT and RET (DX:SI) points to data table:

WORD Drive (0 or 1)

WORD Command:-

0 = Read

1 = Write

2 = Verify

3 = Write With Verify

WORD Starting Logical Sector

WORD Number of Sectors

DWORD Transfer Address (not needed for verify)

RET = status, Number of Sectors field set to

number of sectors transferred.

2.10  Winchester Disk drivers, DEV = 0040H

COM = 0000H    Initialise Drivers

DAT = not used

RET = 0.

COM = 0001H    Set disk status to swapped

DAT = drive number (0 or 1)

RET = 0, or 0FFFFH invalid drive

COM = 0002H   Return Drive type

DAT = drive number (0 or 1)

RET = 3   5  Megabyte Winchester

4   10 Megabyte Winchester

5   20 Megabyte Winchester

80H    No drive fitted

COM =   0003H  Check For Disk Swapped

DAT = drive number (0 or 1)

RET = 0   Disk not swapped

1   Disk swapped

2   No Disk

Note: If RET = 1 is returned, a COM = 1 must

be done, else swapped status will be lost.

COM = 0004H    Get BPB (this should be called

whenever a disk has been swapped).

DEV and RET (DX:SI) points to a data table:

WORD  Drive (0 or 1)

DWORD Pointer to 512 Byte scratch space

RET = return status, and pointer to loaded

BPB in DWORD.

COM = 0005H    Read/Write/Verify/Write with Verify.

DEV and RET (DX:SI) points to data table:

WORD Drive (0 or 1)

WORD Command:

0 = Read

1 = Write

2 = Verify

3 = Write With Verify

WORD Starting Logical Sector

WORD Number of Sectors (128 max)

DWORD Transfer Address

RET = status, also Number of sectors field

set to number of sectors transferred.

2.11 Modem board drivers, DEV = 0041H

Not implemented

2.12 Cache/Graphics/IBM configuration,

DEV = 0042H

COM = 0000H    Return status

DAT = not used

RET = 0, present; 0FFFFH, error

COM = 0001H    Set/Reset Cache on

DAT = 0, reset; 1, set

RET = setting (current if DAT invalid)

COM = 0002H  Set/Reset Graphics/IBM on

DAT = 0, reset,

1, Set Graphics,

2, Set IBM

RET = setting (current if DAT invalid)

*****************    E   N   D   o  f   F   I   L   E  ****************

####################################################