Installing RAM DISK with Intelligence Research Memory Board.

Step 1: Boot system on drive A: using DOS 2.0, BIOS 1.4 or later.

Step 2: Place Intelligence Research disk into drive B:

Step 3: Copy there programs from B: to A:

RDISK.COM

SETRDISK.EXE

OCONFIG.COM

Step 4: From Drive A: run OCONFIG.COM (opens CONFIG.SYS as 'unhidden' file)

Step 5: Now type following:

COPY CON: CONFIG.SYS		<RETURN>

BUFFERS=20			<RETURN>

DEVICE=RDISK.COM		<RETURN>

<CTRL/Z>			<RETURN>

Step 6: From drive A: run SETRDISK.EXE with a parameter for RAM DISK size:

SETRDISK 2

The '2' stands for 2 X 64K memory used by RAM DISK (1=64K,2=128K,etc)

Step 7: Reboot system using modified A: disk. RAM DISK will be installed

to the size specified. (normally drive C:)