GSX Release 1.6.3      8th Mar '85.

-----------------      -----------

For APRICOT PC, XI, F1, Portable and 7220.

-----------------------------------------

RELEASE CONTENTS.

----------------

FILE                               VERSION    DATE     SIZE

----                               -------    ----     ----

1. GRAPHICS.EXE - GDOS                  1.3    12/09/84   8192

2. MOUSE.SYS    - MS MOUSE DRIVER       1.0.3  07/02/85   7941

3. ASSIGN.SYS   - recommendation

4. PC.GSX       - PC/Xi 800*400 mono    1.6.3  28/02/85  19968

5. F1.GSX       - F1 640*256*4 colors   1.6.3   8/03/85  21632

6. F116.GSX     - F1 320*256*16 colors  1.6.3   8/03/85  23424

7. PORTLCD.GSX  - PORT 640*200 LCD mono 1.6.3   8/03/85  20608

8. PORTCOL2.GSX - PORT 640*256*4 cols   1.6.3  28/02/85  22016

9. PORTCOL3.GSX - PORT 640*256*8 cols   1.6.3  28/02/85  23296

10.PORTCOL4.GSX - PORT 640*256*16 cols  1.6.3  28/02/85  23168

11.7220.GSX     - PC/Xi 640*256*16 cols 1.6.3  28/02/85  24704

Changes since release 1.6.2

---------------------------

a. GSX Drivers

-----------

1. Line draw optimised in all drivers other than 7220

where this function is performed in hardware.

2. INPUT LOCATOR request changed if that keyboard only

is requested then the mouse will also work.

3. PRESTEL ops now implemented in the 7220 driver.

4. F1 and PORTABLE drivers now function correctly when using

user defined fill with bars and rectangles.

5. The PORTABLE LCD driver when running on a 2 plane system

will switch off the color video.

6. F1, 40 and 80 column, and PORTABLE LCD have fixed BIT BLAT

problem encountered with GWBASIC.

Changes since release 1.6.1

---------------------------

a. GSX Drivers

-----------

1. No longer neccessary to have MOUSE.SYS loaded to use

redefinable cursors.

2. Circles of radius 1 pixel no longer crashes GSX.

3. Optimised BITBLAT for blocks aligned the same.

4. In certain instances BITBLAT was missing out

one row of pixels.

5. Text on bottom of screen no longer loses bottom

row of pixels.

6. Portable 4 plane circles no longer elliptical.

7. Now possible to write 80 columns of text on F1 and PORTABLE.

8. F_SAMPLE_BUTTONS, opcode 124, now uses correct screen origins.

9. New ops for READ/WRITE pixel, ref Section 12.

Changes since release 1.6.0

---------------------------

a. GSX Drivers

-----------

1. CIRCLES rewritten - now much faster and symetrical.

2. EXCHANGE MOUSE FORM:

(i)  Colour selection changed to new GSX 2.0 spec.

(ii) Bug in returning previous form causing memory

corruption fixed.

b. MOUSE Driver.

------------

1. Clipping of cursor causing screen corruption on PORTABLE

left most edge fixed.

2. Moving cursor to bottom of screen causing occasional system

hangs fixed.

3. The mouse driver will now restore the RS232 port if a serial

mouse is not present.

4. The mouse driver now saves more Z80 SIO registers because

of LOTUS 1-2-3.

5. Now supports 7220.

New Features Over GSX 1.3.

-------------------------

The following new operations conform, wherever possible, to

GSX 2.0 specification, as of 10th Sept 1984.

New operations,

1.   Set user defined line style

2.   Bit block move (*)

3.   Set fill interior style

4.   Set fill perimeter visibility

5.   Exchange fill pattern

6.   Fill rectangle

7.   Exchange mouse form

8.   Show cursor

9.   Hide cursor

10.  Sample mouse button states

11.  Prestel operations (*)

Operations marked (*) are extensions to GSX.

The following pages contain descriptions of the new operations.

1.   Set user defined line style.

---------------------------

This  function  sets the  current  user-defined  line  style

pattern  word in the device driver to the value in the  specified

pattern word, 16 bits.

The  most significant bit (MSB) of the pattern word  is  the

left  most  pixel  displayed.    This  line  style  is  used  for

subsequent polyline operations when the application selects user-

defined line style, index 7.

Input

Contrl(1)  --  Escape opcode = 5

Contrl(2)  --  Number of input vertices = 0

Contrl(6)  --  Opcode = 52

intin(1)   --  User line style

Output

Contrl(3)  --  0

2.   Bit Block Move.

--------------

A  facility has been provided for moving bit aligned  blocks

of  data between the display and memory,  using the standard  GSX

interface.

The routine is an escape operator with function code of 51.

Input

contrl(1)  --  Escape opcode = 5

contrl(2)  --  Number of input vertices = 10

contrl(4)  --  4

contrl(6)  --  Function code = 51

contrl(7)  --  Offset of dest. bitmap

contrl(8)  --  Seg.   of dest. bitmap

contrl(9)  --  Offset of srce. bitmap

contrl(10) --  Seg.   of srce. bitmap

intin(1)   --  Dest. bitmap type

intin(2)   --  Srce. bitmap type

ptsin(1)   --  Dest. origin x

ptsin(2)   --  Dest. origin y

ptsin(3)   --  Srce. origin x

ptsin(4)   --  Srce. origin y

ptsin(5)   --  Extent in x direction

ptsin(6)   --  Extent in y direction

ptsin(7)   --  Dest. bitmap width in NDC units

ptsin(8)   --  Dest. bitmap height in NDC units

ptsin(9)   --  Srce. bitmap width in NDC units

ptsin(10)  --  Srce. bitmap height in NDC units

Output

contrl(3)  --  0

intout(1)  --  0 : No errors detected

1 : Illegal source description

2 : Illegal destination description

3 : Illegal rasterop requested

4 : Illegal extent in x direction

5 : Illegal extent in y direction

The following assumptions are made :-

1. All clipping is done by the application.

2. Source and destination rectangles are the same size.

3. Patterns  are specified  as 8 adjacent byte  with  bits

organised with the most significant bit displayed first.

4. If  an address parameter is not required nothing need  be

passed. An address need not be supplied for the display.

5. All parameters are left unchanged.

Note that source and destination rectangles may overlap.

3.   Set Fill Interior Style.

-----------------------

This   function  sets  the  fill  interior  style  used   in

subsequent polygon fill operations.   If the application requests

an unavailable style, the area is hollow filled.  GSX returns the

selected  style  to  the application.   Hollow  style  fills  the

interior  with  the current background colour,  index  0.   Solid

style fills the area with the currently selected fill colour.

Input

contrl(0)  --  Opcode = 23

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 1

contrl(6)  --  Device handle

intin(0)   --  Request fill interior style

0 - Hollow

1 - Solid

2 - Pattern

3 - Hatch

4 - User defined style

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout = 1

intout(0)  --  Fill interior style selected

4.   Set Fill Perimeter Visibility.

-----------------------------

This  function  turns on or off the outline of a fill  area.

When visible,  the default at Open Workstation for this function,

the  border  of  a fill area is drawn in the  current  fill  area

colour with a solid line.   When invisible,  no outline is drawn.

Any nonzero value of the visibility flag causes the perimeter  to

be drawn.

Input

contrl(0)  --  Opcode = 104

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  length of intin array = 1

contrl(6)  --  Device handle

intin(0)   --  Visibilty flag

zero     - invisible

nonzero  - visible

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array = 1

intout(0)  --  Visibility selected

5.   Exchange Fill Pattern.

---------------------

This function redefines the user-definable fill pattern  and

returns  the  old user-definable fill  pattern  definition.   The

inputs  to this routine are two long pointers.   The first points

to a Fill Pattern Definition Block (FPDB),  which defines the new

fill  pattern.   The  second  points  to  a  FPDB  that  will  be

overwritten with the old fill pattern.   If the second pointer is

zero, the old FPDB is not returned.

The format of a FPDB is shown below.

+--------------------------+

|    Reserved for future   |

|       use, must be 1     |

+--------------------------+

|    Reserved for future   |

|       use, must be 1     |

+--------------------------+

|    Reserved for future   |

|       use, must be 0     |

+--------------------------+

|   16 bits by 16 bits of  |

|        pattern data      |

+--------------------------+

For  the  pattern data,  bit 15 of word 1 is the upper  left

hand  bit of the pattern.   Bit 0 of word 16 is the  lower  right

hand  bit of the pattern.   Bit 0 is the least significant bit of

the word.  A bit value of 1 indicates foreground colour and a bit

value  of  0  the background colour.   The colour  used  for  the

foreground is determined by the current fill area colour index.

The  defined pattern is referenced by the Set Fill Interior

Style function as style 4 and in the Fill Rectangle function.

Input

contrl(0)  --  Opcode = 112

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 0

contrl(6)  --  Device handle

contrl(7-8) -  Double word address of the user defined

fill pattern

contrl(9-10)-  Double word address of the area to copy

the old fill pattern

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array = 0

6.   Fill Rectangle.

--------------

This  function  fills a rectangular area  with  the  pattern

defined  with  the Set Fill Pattern function.   The rectangle  is

filled using the writing mode specified in intin(0).

Input

contrl(0)  --  Opcode = 114

contrl(1)  --  Number of input vertices = 2

contrl(3)  --  Length of intin array = 1

contrl(6)  --  Device handle

contrl(7-8) -  Double word address of the destinations

memory form definition block

(Not implemented, physical display assumed)

intin(0)   --  Writing mode

ptsin(0)   --  X-coordinate of upper left of destination

rectangle in RC/NDC

ptsin(1)   --  Y-coordinate of upper left of destination

rectangle in RC/NDC

ptsin(2)   --  X-coordinate of lower right of destination

rectangle in RC/NDC

ptsin(3)   --  Y-coordinate of lower right of destination

rectangle in RC/NDC

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array = 0

7.   Exchange Mouse Form.

-------------------

This function redefines the cursor pattern displayed  during

locator  input  or any time the cursor is shown (see Show  Cursor

function),  and returns the old cursor definition.  The inputs to

this  routine  are two long paramaters.   The first points  to  a

cursor  definition block which is to be used as the  new  cursor.

The second   points  to a cursor definition block which  will  be

overwritten with the old cursor definition block.   If the second

pointer is zero, the old cursor definition block is not returned.

Format of Cursor Definition Block

+--------------------------------+

|    X-coordinate of hot spot    |

+--------------------------------+

|    Y-coordinate of hot spot    |

+--------------------------------+

|      Reserved, must be 1       |

+--------------------------------+

|      Colour Index              |

+--------------------------------+

|      Reserved, must be 0       |

+--------------------------------+

|  16 by 16 bit of cursor mask   |

+--------------------------------+

|  16 by 16 bit of cursor data   |

+--------------------------------+

For the cursor mask and data,  bit 15 of word 1 is the upper

left  hand bit of the pattern and bit 0 of word 16 is  the  lower

right  bit  of the pattern.   Bit 0 is the least significant  bit

(LSB) of the word.   A bit value of 1 indicates foreground colour

and  a bit value of 0 indicates background  colour.   The  colour

used  for foreground  colour is  determined by the colour index.

The  hotspot is the location of the pixel (relative  to  the

upper  left corner of the cursor) that lies over the pixel  whose

address is returned by the input locator function.

The cursor is drawn as follows,

1.   The data under the cursor is saved so it can be

restored when the cursor moves

2.   The mask is ANDed with the data on the screen

3.   The cursor data is XORed with the result of step 2

and displayed on the screen

Input

contrl(0)  --  Opcode = 111

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 0

contrl(6)  --  Device handle

contrl(7-8) -  Double word address of the new cursor

definition block

contrl(9-10)-  Double word address to receive the old

cursor definition block

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array = 0

8.   Show Cursor.

-----------

This function displays the current cursor. The cursor  moves

on the display surface based on information input from a mouse or

the cursor control keys.

The  Show Cursor function and the Hide Cursor  function  are

closely  related.   Once  the cursor is visable,  a  single  Hide

Cursor  causes the cursor to disappear.  GSX maintains the number

of  times the Hide Cursor function is called.   The  Show  Cursor

function must  be called the same number of times for the  cursor

to reappear.   For example, if the Hide Cursor function is called

four  times,  the Show Cursor function must be called four  times

for the cursor to reappear.

However,  the  Show Cursor function provides a reset flag in

intin(0).   If intin(0) is zero, the cursor appears on the screen

reguardless of the number of Hide Cursor calls.   A nonzero value

for intin(0) effects the Show Cursor function as described in the

previous paragraph.

Input

contrl(0)  --  Opcode = 122

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 1

contrl(6)  --  Device handle

intin(0)   --  Reset flag

0 = ignor number of hide cursor calls

nonzero = normal show cursor behavior

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array

9.   Hide Cursor.

-----------

This  function removes the cursor from the display  surface.

This state is the default condition set at Open Workstation.  The

cursor  can appear in a new position when the  apllication  calls

the  Show Cursor function decause GSX updates the position  based

on information input from a mouse or the cursor control keys.

Refer  to the Show Cursor function for a description of  how

the number of Show Cursor calls effects the Hide Cursor function.

Input

contrl(0)  --  Opcode = 123

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 0

contrl(6)  --  Device handle

Output

contrl(2)  --  Number of output vertices = 0

contrl(4)  --  Length of intout array = 0

10.  Sample Mouse Button State.

-------------------------

This  function  returns  the  current  state  of  the  mouse

buttons.   The  leftmost  mouse button is returned in  the  least

significant bit of the word.   A bit value of 1 indicates the key

is currently depressed, a bit value of 0 indicates the key is up.

This function also  returns the current x,y  position of the

cursor.

Input

contrl(0)  --  Opcode = 124

contrl(1)  --  Number of input vertices = 0

contrl(3)  --  Length of intin array = 0

contrl(6)  --  Device handle

Output

contrl(2)  --  Number of output vertices = 1

contrl(4)  --  Length of intout array = 1

intout(0)  --  Mouse button state

ptsout(0)  --  X position of cursor in NDC/RC units

ptsout(1)  --  Y position of cursor in NDC/RC units

11.  Prestel Operations.

------------------

An escape op is provided to allow the setting of  attributes

for  PRESTEL  text.  The only attribute initially  available   is

double  height.   Colour  is  handled  using  standard  GSX  text

functions.

Input

Contrl(1)  --  Escape opcode = 5

Contrl(2)  --  Number of input vertices = 0

Contrl(6)  --  Op code = 55

intin(1)   --  Set text single height  =  0

Set text double height  =  1

Output

Contrl(3)  --  0

An escape operation is also provided to allow the writing of

PRESTEL  text.   This  operation  depends on  the  standard  font

pointer  refering  to  a  double width font for  40  coloumn mode

PRESTEL.

Input

contrl(1)  --  Escape opcode = 5

contrl(2)  --  Number of input vertices = 1

contrl(4)  --  Number of characters in string

contrl(6)  --  Opcode = 56

intin      --  Word character string = 0

ptsin(1)   --  X-coordinate for start of string

ptsin(2)   --  Y-coordinate for start of string

Output

contrl(3)  --  0

Notes

This  routine assumes that characters require  no  clipping.

The  x,y position for the character is the bottom left corner  of

the character cell NOT the character origin.

The  writing  mode  is always transparent (mode 2)  so  that

PRESTEL  can  use bar for the background and then draw  'OR'  the

character onto this.

12. Read/Write PIXEL.

----------------

An escape op is provided to allow the reading and writing of

an individual pixel. This op is primarily provided for GW-BASIC.

A version of this op will be provided in GSX 2.0.

Input

Control(1)    =  5   ; escape op code

Control(2)    =  1   ; number of input vertices

Control(6)    = 54   ; op code

Input_info(1) = 0    ; READ pixel

= 1    : WRITE pixel

Input_info(2) = value to write (0..15 colour index)

Input_pts(1)  = x coord of pixel

Input_pts(2)  = y coord of pixel

Output

Control(3)    = 0

Output_info(1)= return value for read pixel (0..15 colour index)

APPENDIX.

--------

1.   Colours

The default colours are as follows,

0    Black

1    White

2    Red

3    Green

4    Blue

5    Cyan

6    Yellow

7    Magenta

8    Grey

9    Light Blue

10   Light Green

11   Light Cyan

12   Light Red

13   Light Magenta

14   Brown

15   Low-intensity White

2.   Line styles

1    Solid

2    Long dash

3    Dot

4    Dash, dot

5    Dash

6    Dash, dot, dot

7    User defined style

3.   Fill pattern

0    Hollow

1    Solid

2    Pattern

3    Hatch

4    User defined style

4.    The mouse is supported via a MSDOS loadable device  driver,

MOUSE.SYS. An additional entry is required in CONFIG.SYS

DEVICE=MOUSE.SYS

All APRICOTs' and all mice (MICROSOFT mouse, APRICOT infra-red

and keyboard mice) are supported via a single generic mouse driver.

(N.B. keyboard mice will be supported by all BIOS releases AFTER

BIOS version 2.4)). The mouse driver need only be loaded if a mouse

is physically being used.

5.   Sample mode mouse, as per GSX 1.0 specification

6.   Circles

All radius specifications assume an extent (distance) in the

x-axis.

7.    Using the default ASSIGN.SYS supplied you must copy the

appropiate CRT device driver to DDACRT.SYS before loading GSX.

Alternatively ASSIGN.SYS can be edited to include all of the CRT

drivers, all files with a '.GSX' extension, and the application

open the appropiate driver. The basic philosophy is each system

is assigned 2 driver numbers, the first mono and second colour.

Note the largest driver must be the first driver in ASSIGN.SYS.

Reccomended driver assignments:-

02 7220    .GSX

01 PC      .GSX

03 PORTLCD .GSX

04 PORTCOL2.GSX - this can be replaced with PORTCOL3/4.GSX

05 F1      .GSX

06 F1      .GSX - this can be replaced with F116.GSX