WY

*************************************************************************

DOCUMENTATION FOR ACT ROM BIOS VERSION 1.00

FOR APRICOT PC, APRICOT XI, APRICOT PORTABLE & APRICOT F1

DATE:	9th January 1985	G.Kurth

REV. C  |  change:- error status on get drive & disk type.

REV. D	|  change:- pointer area usage

REV. E	|  change:- floppy driver call modifications

REV. F	|  change:- Winchester board type flag

REV. G	|  change:- Apricot ROM & RAM v3.X BIOS's

REV. H	|  change:- GSX Driver Flag byte

REV. I	|  change:- INT 0EDh

REV. J	|  change:- Floppy Format return status

REV  K  |  change:- AUX. 2 extra calls added. 5/10/84 RJW.

REV. L	|  change:- Floppy driver differences for Apricot BIOS 2.6

REV. M	|  change:- AUX Func 24 - waits for all sent

REV. N	|  change:- Proposed Concurrent CPM screen support

*******************************************************************************

NOTES:	#	- Not implemented on Apricot & Apricot XI BIOS's to v2.4

@	- Not implemented on Apricot & Apricot XI ROM & RAM v3.X BIOS's

%	- Not implemented on Apricot Portable (Rascal A)

$	- Not implemented on Apricot F1 (Rascal C)

*******************************************************************************

*************************************************************************

Extended BIOS Control calls documentation

**************************************************************************

The Control Device allows for the setting of various BIOS parameters by

application programs, and for the ROM/RAM BIOS interface.

Two methods of calling the control device are provided:

INTERRUPT 0FCH  :-

Parameters are passed via registers:

ENTRY:

BX = Device number        (word)

CX = Command              (word)

DX = Data                 (word)

Optionally DX:SI is a double word pointer

to a data table.

EXIT:

AX = Returned data/status (word)

All registers except AX preserved.

FAR CALL TO 0600H :-

Parameters are passed via stack,

Four stack entries are required = Four

Word pointers to parameters in current

Data Segment.

First Parameter pushed:

Word Pointer to Device Number (word)

Second Parameter pushed:

Word Pointer to Command       (word)

Third Parameter pushed:

Word Pointer to Data out      (word)

Fourth Parameter pushed:

Word pointer to Data/Status return

(word)

Optionally the third and fourth parameters

form a double word pointer to a data table

- note that the third parameter is the segment

and the fourth is the offset.

All registers preserved.

***********************************************

*****************************************************************************

DEVICE NUMBERS :-

Valid device numbers are as follows

ASCII   HEX     Device

1       31      Screen Driver

2       32      Keyboard Driver

3       33      MicroScreen Driver

4       34      Serial I/O Driver

5       35      Parallel I/O Driver

6       36      Mouse Driver

7       37      Clock Driver

8       38      Sound Generator / Key click / Bell

9       39      Floppy Disk Drivers

@       40      Winchester Disk Drivers

A       41      Modem Board Drivers

B       42      Cche/Graphics/IBM configuration

**************************************************************************

RETURNED STATUS CODES :-

These codes are returned by any device when an "Init

Device" call (command '0'), or I/O command is issued:

Some status returns set the 0200H bit to indicate

device busy.

0000H   -       Device present & on-line - status OK

8000H	-	Write Protect

8001H	-	Unknown Unit

8002H	-	Not Ready

8003H	-	Unknown Command

8004H	-	CRC Error

8005H	-	Bad Request Length

8006H	-	Seek Error

8007H	-	Unknown Media

8008H	-	Sector Not Found

8009H	-	Printer Out Of Paper

800AH	-	Write Fault

800BH	-	Read Fault

800CH	-	General Failure

8800H	-	Invalid Image Address (Portable Format Only)

FFFFH   -       Device error (also invalid device/command)

***********************************************************************

COMMAND CODES :-

These vary from device to device as follows:-

**********************************************************************

1) Screen Driver, BX = 0031H

============================

CX = 0000H   Initialise Driver

DX = not used

Returns AX = 0000H = ok

%$         CX = 0001H   Get/Set Text/Graphics Display Mode

DX = 0000H = Set Text Mode

DX = 0001H = Set Graphics mode

Returns AX = setting (current if DX invalid)

%$         CX = 0002H   Get/Set Display On

DX = 0000H = Display on

DX = 0001H = Display off

Returns AX = setting (current if DX invalid)

CX = 0003H   *** DO NOT USE ***

#	   CX = 0004H	Print single ASCII character

DL = ASCII character

(the use of INT 0F1H is faster)

Returns AX = 0000H

#	   CX = 0005H	Print ASCII string

DX:SI is a pointer to:

WORD   number of characters to output

DWORD  string pointer

(the use of INT 0F1H is somewhat faster)

Returns AX = 0000H

#	   CX = 0006H	*** DO NOT USE ***

#	   CX = 0007H	*** DO NOT USE ***

#	   CX = 0008H	*** DO NOT USE ***

#	   CX = 0009H	*** DO NOT USE ***

#	   CX = 000AH	*** DO NOT USE ***

#	   CX = 000BH	*** DO NOT USE ***

#	   CX = 000CH	*** DO NOT USE ***

#	   CX = 000DH	*** DO NOT USE ***

#	   CX = 000EH	Write character on default screen.

DH =  X position on the screen (base 0)

DL =  Y     "    "   "    "        "

SI = a word in the same format as the screen image

low byte = ASCII character

high byte = relevant attributes

#	   CX = 000FH	Update default screen. no parameters.

Screen Image memory contains desired characters.

It has the following Format:

1 word for each character, arranged thus:

Lo byte = 8 bit ascii

Hi byte = Hi Nybble = Background colour index

Lo Nybble = Foreground colour index

OR Hi byte = RIUSxxxx

R=reverse video

I=intensity

U=underline

S=strikethrough

CX = 0010H	Print IBM attribute character

DL = ASCII character code

DH = IBM attribute byte

SI = Screen offset (base 0, 1 word per char)

CX = 0011H	Smear IBM attribute character

DX:SI is a pointer to the following table

WORD	ASCII char (low) + IBM attributes (high)

WORD	Screen offset (base 0, 1 word per char)

WORD	Count of characters

CX = 0012H	Copy block of IBM attribute characters to screen

DX:SI is a pointer to the following table

DWORD	Source Segment:Offset

WORD	Screen offset (base 0, 1 word per char)

WORD	Count of characters

CX = 0013H	Move block of characters on screen

DX:SI is a pointer to the following table

WORD	Source Screen offset (base 0, 1 word per char)

WORD	Dest Screen offset (base 0, 1 word per char)

WORD	Count of characters

****************************************

***************************************************************************

2) Keyboard Driver, BX = 0032H

==============================

CX = 0000H   Initialise Driver (To be used only once at boot)

DX = not used

Returns AX = 0000H = ok

AX = 0001H = keyboard missing

CX = 0001H   Get/set Auto-repeat enable

DX = 0000H = Disable auto-repeat

DX = 0001H = Enable auto-repeat

Returns AX = setting (current if DX invalid)

%$         CX = 0002H   Get/set Auto-repeat lead-in delay (20ms intervals)

DX = 0001H - 00FFH (times 20ms)

Returns AX = setting (current if DX invalid)

%$         CX = 0003H   Get/set Auto-repeat rate (20ms intervals)

DX = 0001H - 00FFH (times 20ms)

Returns AX = setting (current if DX invalid)

CX = 0004H   Get/Set keyboard fall-through mode

DX = 0000H = keys interpreted by BIOS

DX = 0001H = Raw up/down codes passed to DOS

Returns AX = setting (current if DX invalid)

NOTE that the infra-red keyboard only produces

downcodes, and no code is generated for SHIFT

or CONTROL keys, downcodes are Apricot compatible.

CX = 0005H   Flush keyboard input buffer

DX = not used

Returns AX = 0000H

CX = 0006H   Reset Keyboard control/shift status

and clear downcode buffer.

DX = not used

Returns AX = 0000H

CX = 0007H   Get/Set HELP key detection in fall-through mode.

DX = 0000H = HELP key terminates fall through mode.

DX = 0001H = HELP key ignored.

Returns AX = setting (current if DX invalid)

CX = 0008H   Place data into the keyboard buffer

DL = ASCII data

Returns AX = 0000H = OK

AX = FFFFH = no room in buffer (bell will sound)

CX = 0009H   Sound the bell

DX = not used

Returns AX = 0000H

CX = 000AH   Return number of keys in keyboard buffer

DX = not used

Returns AX = number of keys in buffer (0 = none)

#	   CX = 000BH	Get Key From Keyboard Buffer (waits if

none available).

DX = not used

Returns AL = ASCII key data.

#	   CX = 000CH	Return next character in queue but do

not extract it (look-ahead).

DX = not used

Returns AX = 00XXH - next character in buffer

AX = 0FFFFH if no characters.

#	   CX = 000DH	Place string in keyboard buffer

DX:SI is pointer to:

WORD	Byte count (80 max. not checked)

DWORD	Pointer to string

Returns AX = 0 if string fitted

#	   CX = 000EH	Get string from Keyboard buffer

DX:SI is pointer to:

WORD	Byte count

DWORD	Pointer to string space

Returns AX = 0

#	   CX = 000FH	Get/Set Keyboard status

DH should contain the AND mask,

and DL the OR mask for the status byte.

Status byte is returned in AL.

0 = off, 1 = active.

bit 0 = Ignore HELP key in raw mode

bit 1 = STOP mode on

bit 2 = CALCULATOR mode on

bit 3 = SHIFT LOCK on

bit 4 = raw mode on (raw downcodes sent direct to DOS)

bit 5 = VOICE LED on

bit 6 = KEYBOARD LOCK on

bit 7 = CAPS LOCK on

E.G. To get the current status without changing any

send DX = 0FF00h ( status is AND'ed with 0FFh, then

OR'ed with 00h - end result is no change).

To set the STOP mode on send DX=0FD02h, etc.

****************************************************************************

3) MicroScreen Driver, BX = 0033H

=================================

%$         CX = 0000H   Initialise Driver

DX = not used

Returns AX = 0

%$         CX = 0001H   Print character (lower 8 bits of data word )

DL = character to print.

Returns AX = 0.

%$         CX = 0002H   Send character straight to MicroScreen/Keyboard

Raw, non-interpreted, output.

DL = raw data to keyboard unit

Returns AX = 0.

****************************************

*****************************************************************************

4) Serial Port Driver, BX = 0034H

=================================

CX = 0000H   Initialise Driver

DX = not used

Returns AX = 0.

CX = 0001H   Transmit Character (lower 8 bits of data word)

DL = Character to transmit

Returns AX = 00000H - all well

AX = 0FFFFH - failed. (or BUSY return).

NOTE: Apricot BIOS 2.6 does not return a fail

but waits until the character can be transmitted.

CX = 0002H   Receive character

DX = not used

Returns AX = 0FFFFH - no character available

AX = 000XXH - received character

CX = 0003H   Update SIO settings - reset Baud rates etc.

As set by calls 0004H to 0009H, 0018h and 0019h.

DX = not used

Returns AX = 0.

CX = 0004H   Get/Set Transmit Baud rate (data = 1-15)

DX = 1 - 15 = baud rate setting

Returns setting in AX (current if DX invalid)

CX = 0005H   Get/Set Receive Baud rate (data = 1-15)

DX = 1 - 15 = baud rate setting

Returns setting in AX (current if DX invalid)

CX = 0006H   Get/Set Transmit bits per character (data = 5-8)

DX = 5 - 8 = bits per character

Returns setting in AX (current if DX invalid)

CX = 0007H   Get/Set Receive bits per character (data = 5-8)

DX = 5 - 8 = bits per character

Returns setting in AX (current if DX invalid)

CX = 0008H   Get/Set number of stop bits per character (data = 1-3)

(data: 1=1, 2=1.5, 3=2 stop bits).

DX = 1 - 3 = stop bit code

Returns setting in AX (current if DX invalid)

CX = 0009H   Get/Set Parity Type (data = 0-2)

(data: 0=none, 1=odd, 2=even).

DX = 0 - 2 = parity type code

Returns setting in AX (current if DX invalid)

CX = 000AH   Get/set Transmit XON/XOFF protocol

DX = 0 = disabled

DX = 1 = enabled

Returns setting in AX (current if DX invalid)

CX = 000BH   Get/set Receive XON/XOFF protocol

DX = 0 = disabled

DX = 1 = enabled

Returns setting in AX (current if DX invalid)

CX = 000CH   Set/reset/get RTS line

DX = 0 = reset

DX = 1 = set

Returns setting in AX (current if DX invalid)

CX = 000DH   Set/reset/get DTR line

DX = 0 = reset

DX = 1 = set

Returns setting in AX (current if DX invalid)

CX = 000EH   Set/reset/get Transmit enable

DX = 0 = disable

DX = 1 = enable

Returns setting in AX (current if DX invalid)

CX = 000FH   Set/reset/get Receive enable

DX = 0 = disable

DX = 1 = enable

Returns setting in AX (current if DX invalid)

CX = 0010H   Get CTS line status

DX = not used

Returns AX = 0 = reset

AX = 1 = set

CX = 0011H   Get DCD line status

DX = not used

Returns AX = 0 = reset

AX = 1 = set

CX = 0012H   Get DSR line status

DX = not used

Returns AX = 0 = reset

AX = 1 = set

CX = 0013H   Set/reset nulls after CR (carriage return)

DX = 0000H to 00FFH nulls (0 = disabled)

Returns setting in AX (current if DX invalid)

CX = 0014H   Set/reset nulls  after FF (form feed)

DX = 0000H to 00FFH nulls (0 = disabled)

NOTE ten times the number of nulls will be sent.

Returns setting in AX (current if DX invalid)

CX = 0015H   Set/reset auto LF after CR

DX = 0 = disable

DX = 1 = enable

Returns setting in AX (current if DX invalid)

CX = 0016H   Enable serial mouse

(Serial receive data generates an INT 0FAh with

the data in AL).

DX = not used

Returns AX = 0.

CX = 0017H   Disable Serial Mouse.

DX = not used

Returns AX = 0.

CX = 0018H   Set/reset RTS/CTS protocol

DX = 0 = disable

DX = 1 = enable

DX = 2 = Auto-enables mode

Returns setting in AX (current if DX invalid)

CX = 0019H   Set/reset DTR/DSR protocol

DX = 0 = disable (& disable auto-enables)

DX = 1 = enable  (& disable auto-enables)

DX = 2 = Auto-enables mode

Note: this call will cause the auto-enables bit to

be set or cleared immediately, without the need

for an "update SIO settings" call (cx=3).

Returns setting in AX (current if DX invalid)

CX =	001AH	Set/Reset External SIO control

DX = 0 = SIO controlled By BIOS

DX = 1 = SIO RS232 channel interrupts

vectored via Int 0F0H.

Returns AL = Base SIO port Address

Order is:	[base+0] = Ch A data

[base+2] = Ch A status

[base+4] = Ch B data

[base+6] = Ch B status

AH = 0 - RS232 on channel A, 1 - channel B.

#	   CX =	001BH	Return next RX character but do not remove

from queue (look-ahead).

DX = not used

Returns AX = 0FFFFH if none available.

AX = 000XXH - RX character

#	   CX =	001CH	Flush Transmit Buffer

DX = not used

Returns AX = 0.

#	   CX =	001DH	Flush receive Buffer

DX = not used

Returns AX = 0.

#	   CX =	001EH	Get receive character status (& reset

error bits).

DX = not used

Returns status in AL as per SIO Read Register 1.

#	   CX =	001FH	Transmit string

DX:SI is a pointer to:

WORD	byte count

DWORD	String pointer

Returns AX = 0 if string transmitted, or

AX = error code.

Byte Count field updated with number transmitted

#	   CX =	0020H	Receive string

DX:SI is a pointer to:

WORD	byte count

DWORD	String space pointer

Returns AX = 0

#          CX = 0021H   set/get receive queue length (512 bytes max)

DX = 1 - 512 = queue length

Returns setting in AX (current if DX invalid)

#          CX = 0022H   set/get transmit queue length (512 bytes max)

DX = 1 - 512 = queue length

Returns setting in AX (current if DX invalid)

#	   CX = 0023H   return number of chars in receive queue, in AX

#	   CX = 0024H   return number of chars in transmit queue, in AX

Does not return 0 unless 'All Sent' bit in SIO is set.

****   ****   ****   ****   ****   ****

Values for Baud rates are as follows:-

1       -       50 baud

2       -       75 baud

3       -       110 baud

4       -       134.5 baud

5       -       150 baud

6       -       300 baud

7       -       600 baud

8       -       1200 baud

9       -       1800 baud

10      -       2400 baud

11      -       3600 baud

12      -       4800 baud

13      -       7200 baud

14      -       9600 baud

15      -       19200 baud

.

Values for stop bits are as follows:-

1       -       one

2       -       one and a half

3       -       two

Values for Parity type are:-

0       -       no parity

1       -       odd parity

2       -       even parity

****************************************

************************************************************************

5) Parallel I/O driver, BX = 0035H

==================================

CX = 0000H   -       Initialise Driver

DX = not used

Returns AX = 0

CX = 0001H   -       Return number of bytes free

in printer output buffer

(if less than 2-3 regard as full)

DX = not used

Returns bytes free in AX.

$          CX = 0002H   -       Get/Set FAULT line status detect

DX = 0 - disable

DX = 1 - enable

Returns setting in AX (current if DX invalid)

$          CX = 0003H   -       Get/Set SELECT line status detect

DX = 0 - disable

DX = 1 - enable

Returns setting in AX (current if DX invalid)

$          CX = 0004H   -       Get/Set PAPER ERROR line status detect

DX = 0 - disable

DX = 1 - enable

Returns setting in AX (current if DX invalid)

CX = 0005H   -       Get/Set Auto LF after CR enable

DX = 0 - disable

DX = 1 - enable

Returns setting in AX (current if DX invalid)

CX = 0006H   -       Get/Set Serial/Parallel PRN: device output

DX = 0 - Parallel PRN:

DX = 1 - Serial PRN:

Returns setting in AX (current if DX invalid)

CX = 0007H   -       Print character on device set

by the above call (Serial or Parallel)

DL = character

Returns AX = 0 -OK, or

AX = error code

#	   CX =	0008H	-	Flush printer output buffer

DX = not used

Returns AX = 0.

#	   CX =	0009H	-	Transmit string (serial or parallel)

DX:SI is a pointer to:

WORD	byte count

DWORD	String pointer

Returns AX = 0 if string transmitted, or

AX = error code.

#	   CX =	000AH	-	Return output status

DX = not used

Returns AX = 0 -ok, or

AX = Printer busy, or no paper code.

***************************************************************************

6) Mouse driver, BX = 0036H

===========================

Not internally implemented

****************************************

**************************************************************************

7) Clock Driver, BX = 0037H

===========================

#	   CX =	0000H	-	Initialise Clock Driver

Resets time to 0:00:00 1-1-80

DX = not used

Returns AX = 0

#	   CX =	0001H	-	Set Date/Time

DX:SI points to data table:

WORD	Days Since 1-1-80

BYTE	Minutes

BYTE	Hours

BYTE	Hundredths Of Seconds

BYTE	Seconds

Returns AX = 0.

#	   CX =	0002H	-	Get Date/Time

DX:SI points to 6-BYTE data space

for above table.

Returns AX = 0.

****************************************

***************************************************************************

8) Sound Generator / Key click / bell, BX = 0038H

=================================================

CX = 0000H   -       Initialise Sound Driver

DX = not used

Returns AX = 0.

CX = 0001H   -       Get/Set Key feedback click volume

DX = 0 - 15 ( 0 = max, 15 = min)

Returns setting in AX (current if DX invalid)

CX = 0002H   -       Get/Set bell volume

DX = 0 - 15 ( 0 = max, 15 = min)

Returns setting in AX (current if DX invalid)

$          CX = 0003H   -       Set channel 2 volume

DX = 0 - 15 ( 0 = max, 15 = min)

Returns setting in AX (current if DX invalid)

$          CX = 0004H   -       Set channel 3 volume

DX = 0 - 15 ( 0 = max, 15 = min)

Returns setting in AX (current if DX invalid)

$          CX = 0005H   -       Set channel 2 frequency

DX = 0 - 1023 ( 0 = max, 1023 = min)

Returns setting in AX (current if DX invalid)

$          CX = 0006H   -       Set channel 3 frequency

DX = 0 - 1023 ( 0 = max, 1023 = min)

Returns setting in AX (current if DX invalid)

CX = 0007H   -       Set bell frequency

DX = 0 - 1023 ( 0 = max, 1023 = min)

Returns setting in AX (current if DX invalid)

CX = 0008H   -       Set bell duration

DX = 1 - 255 (times 20ms)

Returns setting in AX (current if DX invalid)

#	   CX =	0009H	-	Sound Bell

DX = not used

Returns AX = 0

#	   CX =	000AH	-	Sound Click

DX = not used

Returns AX = 0

****************************************

******************************************************************************

9) Floppy Disk drivers, BX = 0039H

==================================

CX = 0000H   Initialise Drivers (No action performed).

DX = not used

Returns AX = 0

CX = 0001H   Set Drive number for format

DX = Drive (0 or 1)		 *** SEE NOTE BELOW ***

Returns AX = 0 or 'UNKNOWN UNIT'/0FFFFh if invalid

CX = 0002H   Set Segment address of Track image for format

DX = segment address of image

Returns AX = 0

CX = 0003H   Set Offset address of Track image for format

DX = offset address of image

Returns AX = 0

CX = 0004H   Format Disk     (floppy only)

DX = 0       70 track SS

DX = 1       80 track SS

DX = 2       80 track DS

Returns AX = Error status, or 0 = ok.

NOTE: On the Portable a return of 8800H indicates

that the supplied format image was at an invalid

address - it is valid to retry with an alternative

image location.

CX = 0005H   Return status of requested disk

APRICOT COMPATIBLE VERSION - WILL NOT BE SUPPORTED

ON FUTURE RELEASES.

DX = drive (0 to 3)		 *** SEE NOTE BELOW ***

THESE ARE LOGICAL DRIVE NUMBERS AND COVER BOTH THE

FLOPPY AND WINCHESTER DRIVES.

Returns AX = status as WD279X type 1 status

or AX = 'UNKNOWN UNIT'/0FFFFh if invalid.

CX = 0006H   Set disk status to swapped

DX = Drive (0 or 1)		 *** SEE NOTE BELOW ***

Returns AX = 0, or 'UNKNOWN UNIT'/0FFFFh if invalid.

CX = 0007H   Return disk type

DX = drive number (0 or 1)	 *** SEE NOTE BELOW ***

Returns AX = type :  0       70 track SS floppy

1       80 track SS floppy

2       80 track DS floppy

Note: Returns status for  "unknown unit"  if no

drive fitted.

CX = 0008H   Return drive type

DX = drive number (0 or 1)	 *** SEE NOTE BELOW ***

Returns AX = type :  0       70 track SS floppy

1       80 track SS floppy

2       80 track DS floppy

Note: Returns status for  "unknown unit"  if no

drive fitted.

#	   CX =	0009H	Check For Disk Swapped

DX = drive number (0 or 1)	 *** SEE NOTE BELOW ***

Returns AX =	0	Disk not swapped

1	Disk swapped

2	No Disk

or 'UNKNOWN UNIT'/0FFFFh if invalid.

#	   CX =	000AH	Get BPB (this should be called

whenever a disk has been swapped).

DX:SI points to a data table:

WORD	Drive (0 or 1)	 *** SEE NOTE BELOW ***

DWORD	Pointer to 512 Byte scratch space

Returns status in AX, and pointer to loaded

BPB in the DWORD pointer.

#	   CX =	000BH	Read/Write/Verify/Write with Verify.

DX:SI points to data table:

WORD Drive (0 or 1)	 *** SEE NOTE BELOW ***

WORD Command

0 = Read

1 = Write

2 = Verify

3 = Write With Verify

WORD Starting Logical Sector

WORD Number of Sectors

DWORD Transfer Address (not needed for verify)

Returns AX = status, Number of sectors field

set to number of sectors transferred.

#          CX = 000CH   Return status of requested disk

DX = Drive (0 or 1)		*** SEE NOTE BELOW ***

Returns AX = status as WD279X type 1 status

or AX = 'UNKNOWN UNIT'/0FFFFh if invalid.

Note: This call should be used in preference to call 5.

**** NOTE ****	Floppy Driver calls which include a drive number:-

Differences between Apricot BIOS 2.6 and all ROM BIOS's/BIOS 3.X.

MACHINE CONFIGURATION		BIOS 2.6		ROM BIOS's/BIOS 3.X

=====================		========		===================

(CALLS 1,5,6,7 & 8)   DRIVE 0     1     2     3     DRIVE 0     1     2     3

--------------------------    --------------------------

SINGLE FLOPPY		  FLPY1   X     X     X         FLPY1   X     X     X

DUAL FLOPPY		  FLPY1 FLPY2   X     X         FLPY1 FLPY2   X     X

1 FLOPPY & 1 WINI	  WINI1 FLPY1   X     X         FLPY1   X     X     X

2 FLOPPY & 1 WINI	  WINI1 FLPY1 FLPY2   X         FLPY1 FLPY2   X     X

1 FLOPPY & 2 WINI	  WINI1 WINI2 FLPY1   X         FLPY1   X     X     X

2 FLOPPY & 2 WINI	  WINI1 WINI2 FLPY1 FLPY2       FLPY1 FLPY2   X     X

CALLS 9,A,B & C For all BIOS's are as shown above for 'ROM BIOS's/BIOS 3.X'

CALL 5 For all BIOS's is as shown above for 'BIOS 2.6'

CALLS 7 & 8 Will also return winchester drive types in BIOS 2.6 as follows:

'3' = 2 head,   '4' = 4 head,  '5' = 8 head

******************************************************************************

10) Winchester Disk drivers, BX = 0040H

=======================================

#          CX = 0000H   Initialise Drivers

DX = not used

Returns AX = 0.

#          CX = 0001H   Set disk status to swapped

DX = drive number (0 or 1)

Returns AX = 0, or 0FFFFH invalid drive

#          CX = 0002H   Return Drive type

DX = drive number (0 or 1)

Returns AX = type : 3	2 head Winchester

4	4 head Winchester

5	8 head Winchester

Note: Returns status for  "unknown unit"  if no

drive fitted.

#	   CX =	0003H	Check For Disk Swapped

DX = drive number (0 or 1)

Returns AX =	0	Disk not swapped

1	Disk swapped

2	No Disk

#	   CX =	0004H	Get BPB (this should be called

whenever a disk has been swapped).

DX:SI points to a data table:

WORD	Drive (0 or 1)

DWORD	Pointer to 512 Byte scratch space

Returns status in AX, and pointer to loaded

BPB in the DWORD pointer.

#	   CX =	0005H	Read/Write/Verify/Write with Verify.

DX:SI points to data table:

WORD Drive (0 or 1)

WORD Command

0 = Read

1 = Write

2 = Verify

3 = Write With Verify

WORD Starting Logical Sector

WORD Number of Sectors

DWORD Transfer Address

Returns AX = status, Number of sectors field

set to number of sectors transferred.

*******************************************************************************

11) Modem board drivers, BX = 0041H

===================================

Not internally implemented

******************************************************************************

12) Cache/Graphics/IBM configuration, BX = 0042H

================================================

@%$        CX = 0000H	Return status

DX = not used

Returns AX = 0 - present, 0FFFFH - error

@%$        CX = 0001H 	Set/reset Cache on

DX = 0 - reset, 1 - set

Returns setting in AX (current if DX invalid)

@%$        CX = 0002H  Set/reset Graphics/IBM on

DX = 0 - reset, 1 - set Graphics, 2 - set IBM

Returns setting in AX (current if DX invalid)

*************************************************************************

POINTERS Documentation - Fixed format data area

*************************************************************************

This is the 1K area of RAM between 400H and 7FFH

Address Length  Set by  Use

======= ======  ======  ===

400H    1       BOOT    Boot PROM version number

401H	1	BOOT	Machine type

0 = Apricot & Apricot XI

1 = Apricot Portable (Rascal A)

2 = Apricot F1 & F1e (Rascal C)

402H    2       BOOT    Random Access Memory Size (in paragraphs)

404H    4       BOOT    Cold/Warm bootstrap mark (5678H,1234H)

408H    2       BOOT    Drive booted from

0000H = Floppy 0	0001H = Floppy 1

0002H = Winchester 1	0003H = Winchester 2

40AH    4       BOOT    Pointer to loaded boot disk header sector

40EH    4       BOOT    Not Used

412H    1       BOOT    8089 present flag (01=present, 00=missing) [not used on F1]

[Portable: Video flag 0= LCD, 1= 2 plane, 2= 4 plane]

413H	1	BOOT	Winchester board type (00=up to V7, 01=V9[dual])

414H    1       BOOT    Winchester type:0 = No Winchester

3 = 2 head Winchester

4 = 4 head Winchester

5 = 8 head Winchester

415H    1       BOOT    Floppy type:    0 = 70 track SS

1 = 80 track SS

2 = 80 track DS

416H	1	BOOT	Number of Floppy Drives

417H	1	BOOT	Number of Winchester Drives

418H	4	BOOT	Pointer to Floppy BPB array Table

41CH	4	BOOT	Pointer to Winchester BPB array Table

420H    2       BOOT    Boot Diagnostic results

Single error number as a word (first error found)

422H	2	BOOT	Paragraph address of start of user code area

(for use by BIOS's loaded from disk).

424H	4	BOOT	ROM BIOS Stack Segment/offset (for use by interrupt routines)

500H    8       BOOT    8089 IOP channel control block - channel 1 (disk)

508H    8       BOOT    8089 IOP channel control block - channel 2 (expansion)

600H    5       BOOT    Long jump to BIOS Control Device

606H	1	APP	GSX Driver Flag - For Activity/Actex etc.

610H    112     USER    Reserved for MBASIC/GSX interface

700H    4       BOOT    Pointer to internal BIOS config table

704H    2       BOOT    Length of internal BIOS config table in bytes

706H    4       BOOT/

USER    Pointer to active character font

70AH    2       BOOT/

USER    Length of active character font in bytes

70CH    4       BOOT/

USER    Pointer to master character font

710H    2       BOOT/

USER    Length of master font in bytes

712H    4       BOOT/

USER    Pointer to active keyboard tables

716H    2       BOOT/

USER    Length of active keyboard tables

718H    4       BOOT    Pointer to internal keyboard tables

71CH    2       BOOT    Length of internal keyboard tables

71EH    4       BOOT/

USER    Pointer to MicroScreen Character translate table

722H	4	BOOT	Pointer to write only register copy table

726H	4	BOOT	Pointer to ASCII colour screen image

72AH	4	BOOT	Pointer to ASCII LCD screen image (portable only)

780H	4	APP	Voice software pointer area

NOTE:	Some of these pointers are either not applicable, or not used

on some machines.

NOTE:   'USER' indicates that these parameters can not only be accessed

by user programs, but can also be modified.

All other parameters can be accessed, BUT NOT MODIFIED.

***********************************************************************

ACT standard reserved interrupts 	14-sep-84 GK

These are the interrupts reserved by ACT for software

Functions within the ROM BIOS.

**********************************************************************

ACT Reserves interrupts 0EEH to 0FFH for ACT use.

Those currently implemented are:

INT	0FFH	-	Clock Interrupt - every 20ms

SOURCE:	ROM BIOS

INPUT:	none

OUTPUT:	none

SS, DS, ES must be preserved,

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0FEH	-	Exec interrupt

SOURCE:	Application

INPUT:	none

OUTPUT:	AX = 0FFFFH

INT	0FDH	-	Spooler Interrupt

SOURCE:	Application

INPUT:	none

OUTPUT:	AX = 0FFFFH

INT	0FCH	-	BIOS Control interrupt

SOURCE:	RAM BIOS & Application

INPUT:	AX,BX,CX,DX,SI

OUTPUT:	AX - Other registers preserved.

INT	0FBH	-	Mouse Interrupt 2

SOURCE:	Application

INPUT:	none

OUTPUT:	AX = 0FFFFH

INT	0FAH	-	Mouse Interrupt 1 - MicroSoft serial Mouse

SOURCE:	ROM BIOS

INPUT:	AL = Mouse Data Byte

OUTPUT:	SS, DS, ES must be preserved

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0F9H	-	Keyboard Interrupt

SOURCE:	ROM BIOS

INPUT:	AL = BL = Decoded Key

AH = BH = Count of characters in string

OUTPUT:	AX = 0FFFFH - pass key to DOS queue

AX = 00000H - Ignore key

BL = Translated Character

All other registers must be preserved,

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0F8H	-	Voice Interrupt 3

SOURCE:	Application

INPUT:	none

OUTPUT:	none

INT	0F7H	-	Voice Interrupt 2

SOURCE:	ROM BIOS

INPUT:	AX = Voice Key Code Packet

OUTPUT:	none

SS, DS, ES must be preserved,

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0F6H	-	Voice Interrupt 1

SOURCE:	Application

INPUT:	none

OUTPUT:	none

INT	0F5H	-	Mouse Interrupt 3 - I/R & Apricot Keyboard Mouse

SOURCE:	ROM BIOS

INPUT:	AX = Mouse packet

bits 0-7 = X or Y data

bit 8    = 1 -Y packet, 0 -X packet

bit 9    = Switch 1 (1=on)

bit 10   = Switch 2 (1=on)

bit 11   = 1

bits 12-15 = undefined

OUTPUT: None

All registers must be preserved,

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0F4H	-	External Interrupt Setup

Replace External interrupt vectors and enable int.

SOURCE:	Application

INPUT:	AL = 0 - Set External Interrupt 2 (winchester)

AL = 1 - Set External Interrupt 3 (general)

BX = Vector Offset Word

CX = Vector Segment Word

OUTPUT:	BX = Old Vector Offset Word

CX = Old Vector Segment Word

INT	0F3H	-	Voice Interrupt 4

SOURCE:	Application

INPUT:	n/a

OUTPUT:	n/a

INT	0F2H	-	Special 25th line output

(For calculator & specialist applications)

SOURCE:	ROM BIOS & Application

INPUT:	CX = command, AL = data

OUTPUT:	none, all registers preserved

Commands:

CX=0, init line 25. AL=ID code (below)

CX=1, rest line 25. AL=ID code (below)

CX=2, print character in AL

CX=3, set cursor to position AL (0-79)

ID codes: 0=voice, 1=calc.

INT	0F1H	-	Screen Output (also provided by INT 29H)

SOURCE:	RAM BIOS & Application

INPUT:	AL = character

OUTPUT:	none, all registers preserved.

INT	0F0H	-	SIO control Interrupt

SOURCE:	ROM BIOS

INPUT:	AL = SIO interrupt vector

0  = Ch B Tx buffer empty

2  = Ch B ext/status int.

4  = Ch B Rx ready

6  = Ch B Special Rx

8  = Ch A Tx buffer empty

10 = Ch A ext/status int.

12 = Ch A Rx ready

14 = Ch A Special Rx

OUTPUT:	none

SS, DS, ES must be preserved,

Stack must not be changed if interrupts

are enabled, or a call to the ROM BIOS is to

be made.

INT	0EFH	-	RESERVED FOR USE BY GEM

INT	0EEH	-	Send Non-specific EOI to PIC, or dummy

'RETI' sequence to Z80 chips.

SOURCE:	Application Hardware Interrupt Handler

INPUT:	none

OUTPUT:	none

INT	0EDH	-	RESERVED - Standard Interrupt Vector Reference

******************************************************************************

ACT Hardware Specific interrupts.

=================================

This document describes the interrupts generated by hardware

on the ACT Apricot family.

1)	Apricot & Apricot XI

============================

INTERRUPT CONTROLLER	: Intel 8259A PIC.

PORTS:	00H = EOI port, 02H = Mask register.

METHOD of enabling interrupt:

Read Mask register (port 02H), clear required bit,

and write new value to mask register.

MODE of interrupt return:

Send EOI (20h) to EOI port (00H).

SPECIALS:

Z80 SIO

Interrupt vector obtained by performing a LOCK IN from

Port 60H.

The SIO must be sent a return from interrupt command

before an EOI is sent to the PIC.

STANDARD INTERRUPTS:

INT 50H	- 8089 Channel 1 interrupt 			(IMR bit 0)

INT 51H	- 8089 Channel 2 interrupt 			(IMR bit 1)

INT 52H	- Expansion interrupt 1 (for winchester) 	(IMR bit 2)

INT 53H - Expansion interrupt 2 			(IMR bit 3)

INT 54H - Floppy Disk Controller Interrupt 		(IMR bit 4)

INT 55H	- Z80 SIO interrupt (see SPECIALS above)	(IMR bit 5)

INT 56H	- Programmable Interval Timer (system clock)	(IMR bit 6)

INT 57H - 8087 Numeric Data Processor interrupt		(IMR bit 7)

******************************************************************************

2)	Apricot Portable (Rascal A)

===================================

INTERRUPT CONTROLLER	: Intel 8259A PIC.

PORTS:	68H = EOI port, 6AH = Mask register.

METHOD of enabling interrupt:

Read Mask register (port 6AH), clear required bit,

and write new value to mask register.

MODE of interrupt return:

Send EOI (20h) to EOI port (68H).

SPECIALS:

Z80 SIO

Interrupt vector obtained by performing the following

sequence:

Write a 2 to SIO channel B command port (1Eh)

Read vector from SIO channel B status port (1Eh)

The SIO must be sent a return from interrupt command

before an EOI is sent to the PIC.

STANDARD INTERRUPTS:

INT 50H	- Programmable Interval Timer (system clock)	(IMR bit 0)

INT 51H	- Floppy Disk Controller Interrupt		(IMR bit 1)

INT 52H	- Voice controller interrupt		 	(IMR bit 2)

INT 53H - Expansion Interrupt 1 (winchester)		(IMR bit 3)

INT 54H - Z80 SIO interrupt (see SPECIALS above)	(IMR bit 4)

INT 55H	- Expansion Interrupt 2				(IMR bit 5)

INT 56H	- Parallel Printer Interrupt			(IMR bit 6)

INT 57H - DMA controller Interrupt			(IMR bit 7)

******************************************************************************

2)	Apricot F1 (Rascal C)

=============================

INTERRUPT CONTROLLER	: None

Z80 SIO & CTC generate automatic vectored

interrupts.

FDC generates NMI.

METHOD of enabling interrupt:

Enable required interrupt in SIO or CTC.

MODE of interrupt return:

Create Dummy Z80 'RETI' sequence:

Write 0EDH, then 04DH to port 30h,

SPECIALS:

FDC interrupts cannot be masked.

STANDARD INTERRUPTS:

INT 02H	- Floppy disk controller interrupt (NMI)

INT 50H	- SIO ch B TX buffer empty	RS232

INT 52H	- SIO ch B external/status	RS232

INT 54H - SIO ch B RX ready		RS232

INT 56H - SIO ch B special RX status	RS232

INT 58H	- SIO ch A TX buffer empty	SOUND

INT 5AH	- SIO ch A external/status	PRINTER

INT 5CH - SIO ch A RX ready		KEYBOARD

INT 5EH - SIO ch A special RX status	KEYBOARD

INT 60H	- CTC ch 0 	EXPANSION INTERRUPT

INT 62H - CTC ch 1	RS232 BAUD RATE

INT 64H	- CTC ch 2	SOUND FREQUENCY

INT 66H	- CTC ch 3	SYSTEM CLOCK (20ms)

***************************************************************************

REGISTER COPY TABLE FORMATS

===========================

This document describes the format of the Write-only register

copy tables held by the ROM BIOS and made available to the user

through a pointer at absolute address 722H.

1) 	Apricot & Apricot XI  ( @ )

===================================

Offset	size	register

------	----	--------

00h	byte	SIO channel A write register 1	RS232

01h	byte	SIO channel A write register 3	RS232

02h	byte	SIO channel A write register 4	RS232

03h	byte	SIO channel A write register 5	RS232

04h	byte	SIO channel B write register 1	KEYBOARD

05h	byte	SIO channel B write register 3	KEYBOARD

06h	byte	SIO channel B write register 4	KEYBOARD

07h	byte	SIO channel B write register 5	KEYBOARD

2)	Apricot Portable (Rascal A)

===================================

Offset	size	register

------	----	--------

00h	byte	SIO channel A write register 1	RS232

01h	byte	SIO channel A write register 3	RS232

02h	byte	SIO channel A write register 4	RS232

03h	byte	SIO channel A write register 5	RS232

04h	byte	SIO channel B write register 1	KEYBOARD

05h	byte	SIO channel B write register 3	KEYBOARD

06h	byte	SIO channel B write register 4	KEYBOARD

07h	byte	SIO channel B write register 5	KEYBOARD

08h	word	LED / Video control port copy

0Ah	byte	LCD voltage control port copy

3)	Apricot F1 (Rascal C)

=============================

Offset	size	register

------	----	--------

00h	byte	SIO channel B write register 1	RS232

01h	byte	SIO channel B write register 3	RS232

02h	byte	SIO channel B write register 4	RS232

03h	byte	SIO channel B write register 5	RS232

04h	byte	SIO channel A write register 1	KEYBOARD

05h	byte	SIO channel A write register 3	KEYBOARD

06h	byte	SIO channel A write register 4	KEYBOARD

07h	byte	SIO channel A write register 5	KEYBOARD

$