----------------------------------------------------------------------

EPSON.COM       Setup Utility for Epson Printers

Douglas Ritari, modified by Guy Gordon

----------------------------------------------------------------------

The program allows you to send any of eight special commands to the

Epson MX80 to set the printer into desired modes.

A second option allows the entry of hex codes on the command line, which

are then translated into binary and sent to the printer.  This way you

can send any escape code to any printer, though it is less convenient than

using the eight special codes.

Format

----------------------------------------------------------------------

EPSON [E] [D] [I] [W] [C] [T] [R] [U] [/xx/]

The command options are:

E ... Emphasized printing (characters are thicker than usual)

D ... Double-strike printing

I ... Italics

W ... Wide character printing (5 chars per inch)

C ... Compressed printing (17.16 chars per inch)

T ... Top of Form (Form Feed or top of page)

R ... Restes the printer to the power-on state

U ... Unidirectional printing

/xx/ - Hexadecimal input mode.  Send code xx to printer.

Usage

----------------------------------------------------------------------

A typical command may read "EPSON E T" to set Emphasized printing and go

to the top of the next page.

The command "EPSON" with no parameters will set the printer for double-strike,

emphasized.

The command "EPSON /1B38/ disables the paper-out sensor, since the Epson

manual says an ESC-8 will do this function: '1B' in hex is escape, '38' hex

is the character "8".

#######################################